###

___abc (Automatic Baseline Correction)_________________________________

Description: Use abc to perform an automatic baseline correction.

The abc command automatically selects data point to

pull to the new baseline.  (To choose your own data

points, use the base command.)

Returns:     1

Arguments:   None.

Example:     load "co.abs"  /* Load file into memory. */

abc            /* Correct the baseline. */

show           /* Display the spectrum. */

###

___abs2ras (Absorbance To Transmittance)_______________________________

Description: Use to convert absorbance data (the current sample) to

transmittance data.

Arguments:   None.

Returns:     1

Example:     load "co.abs"  /* Load file into memory. */

abs2ras        /* Convert data to transmittance. */

show           /* Display the transmittance data. */

###

___acos________________________________________________________________

Description: Use to compute the arc cosine of the specified value.

Arguments:   Value (in radians) for which to find arc cosine (int or

float).

Example:     y = acos 0   /* Place arc cosine of 0 in y */

/* register. */

x = acos z   /* Place arc cosine of value */

/* in register z into register x. */

###

___add_________________________________________________________________

Description: Use to add data to the data in the memory.  The

resolution of the data you add must be the same

as the resolution of the data in memory.

Arguments:   Name of file you wish to add (string).

Example:     load "pure1.abs"       /* Load file into memory. */

add "pure2.abs"        /* Add pure2 to pure1. */

title "pure1 + pure2"  /* Assign title to result. */

save "puresum.abs"     /* Save the result. */

show                   /* Display the data. */

###

___add2Lib_____________________________________________________________

Description: Add a data file to a user library.

Arguments:   Name of library (string), name of data file to

add (string).  NOTE: If you do not include a data

file name, the current sample is added.

Returns:     1 if successful, 0 if unsuccessful (int).

Example:     load "pure1.abs"   /* Load file into memory. */

add2lib "purelib"  /* Add sample to library. */

add2lib "purelib" "pure2.abs"

/* Add pure2.abs to library. */

###

___append______________________________________________________________

Description: Use to move to the end of a text or binary file that

is currently open for input and output.  NOTE: To avoid

overwriting text or data in an open the file, be sure

to use append before writing to that file.

Arguments:   Number assigned to open file (int).

Example:     fopen 1 "file1"  /* Open file1 for read and write. */

append 1         /* Move to the end of file1. */

fprint 1 "Place this at end of file1"

/* Append text to the file end. */

fclose 1         /* Close the file. */

###

___ArrayTodata_________________________________________________________

Description: Use to find out the wavenumber value for a given array

position.

Arguments:   Array position (int), array to use (variable).

Returns:     Wavenumber associated with position (float).

Example:     load co.abs  /* Load file into memory. */

x = arrayTodata 10 sample:data

/* Get wavenumber at position 10 */

/* and place value in register x. */

print x      /* Print the value to the screen. */

###

___asin________________________________________________________________

Description:  Use to compute the arc sine of the specified value.

Arguments:    Value (in radians) for which you want the sine (int

or float).

Example:      y = asin 0   /* Place arc sine of 0 in y */

/* register. */

x = asin z   /* Place arc sine of value stored */

/* in z register into x register. */

###

___ask_________________________________________________________________

Description: Use to display a yes or no question and wait for a

response.

Arguments:   (optional)  Text of question to display (string).

Returns:     1 if y is typed, 0 if y is not typed (int).  Also, the

return value is stored in the z register.

Example:     float array tmpdata  /* Declare a float array. */

load "co.abs"        /* Load file into memory. */

ask "This macro alters the sample data... Continue?"

if (z == 0)  /* If answer is not y... */

exit        /* Exit the macro. Otherwise, continue. */

tmpdata:data = sample:data

/* Assign sample data to the array */

/* named tmpdata. */

tmpdata:firstx = 400.0  /* Set starting wavenumber. */

tmpdata:lastx = 4000.0  /* Set ending wavenumber. */

sample:data = tmpdata:data

/* Assign tmpdata to be the sample. */

autoscale    /* Display sample with new range. */

###

___atan________________________________________________________________

Description: Use to compute the arc tangent of the specified value.

Arguments:   The value (in radians) for which to calculate the arc

tangent (int or float).

Example:     y = atan 0   /* Place arc tangent of 0 in */

/* register y. */

x = atan z   /* Place arc tangent of value in */

/* register z into register x. */

###

___autoScale___________________________________________________________

Description: Use to rescale sample data to the data limits.

(Autoscale does not undo any data manipulations.)

Arguments:   None.

Example:     load "pure1.abs"          /* Load file into memory. */

expand 3000 3500 0.5 1.5  /* Expand the display. */

show                      /* Show the results. */

autoscale                 /* Rescale data display. */

###

___autoScaleX___________________________________________________________

Description: Use to rescale sample data to the data limits, while

preserving the current y axis display limits.

Arguments:   None.

Example:     load "pure1.abs"          /* Load file into memory. */

expand 3000 3500 0.5 1.5  /* Expand the display. */

show                      /* Show the results. */

autoscalex                /* Rescale x axis of data */

/* display while leaving */

/* y axis unchanged.)

###

___autoScaleY___________________________________________________________

Description: Use to rescale sample data to the data limits, while

preserving the current x axis display limits.

Arguments:   None.

Example:     load "pure1.abs"          /* Load file into memory. */

expand 3000 3500 0.5 1.5  /* Expand the display. */

show                      /* Show the results. */

autoscaley                /* Rescale y axis of data */

/* display while leaving */

/* x axis unchanged.)

###

___average_____________________________________________________________

Description: Use to average a data file you specify with the sample

data loaded in memory.

Arguments:   Name of file to average (string) with sample data.

Example:     load "pure.abs"      /* Load file into memory. */

average "pure2.abs"  /* Average data in pure.abs */

/* and pure2.abs files. */

show                 /* Display the result. */

###

___background__________________________________________________________

Description: Background is an object that contains methods you can

set and execute to load, save and show a current

background data file.

Methods:     Methods for the background object are:

load:          Use to load a background data file to serve as the

current background data file.  The loaded file

must be one of these data types: .igm, .big, .sbm

or .bkg.

Arguments:     The name of the background data file to load

(string).

Syntax:        background:load pure.bkg

show:          Use to display the current background data file

in a data window.

Arguments:     None.

Syntax:        background:show

save:          Use to save the current background data file.

Arguments:     The name of the background data file to save

(string).

Syntax:        background:save pure2.bkg

###

___base________________________________________________________________

Description: Use to correct the baseline at specified wavenumbers.

Arguments:   At least 2, and up to 19, wavenumbers (float).

Example:     load "co.abs"        /* Load file into memory. */

base 2250 2100 2050  /* Correct baseline at these */

/* wavenumbers. */

show                 /* Display the result. */

###

___basename____________________________________________________________

Description: Use to remove the filename extension from a file name.

Arguments:   File name, including filename extension (string).

Returns:     The filename, with no filename extension (string).

Example:     load "pure.abs"          /* Load file into memory. */

x = basename "pure.abs"  /* Assign filename, or */

/* basename, to register x. */

title x            /* Set title of current sample to */

/* the string in register x. */

###

___beep________________________________________________________________

Description: Use to make the computer to beep.

Arguments:   None.

Example:     beep  /* Beep once. */

beep  /* Beep a second time. */

ask "Do you want to initialize the bench?"

/* Ask question and wait for answer. */

###

___bkg_________________________________________________________________

Description: Use to collect background data.

Arguments:   None.

Example:     ask "Would you like to collect a background?"

if (z == 1)  /* If answer is y... */

bkg         /* Collect background data. */

###

___boxcar______________________________________________________________

Description: Use to smooth data using the boxcar method, a straight

running average.

Arguments:   Any odd number to specify the number of points to

average (int).

Example:     load "co.abs"  /* Load file into memory. */

boxcar 9       /* Perform boxcar smooth. */

show           /* Display the results. */

###

___buildString_________________________________________________________

Description: Use to create a character string from any set of

values, including values in variables and registers.

For example, use buildString to create text for output,

text for the ask command, or a string for the

execstring command.

Arguments:   Any legal literal values, variables, or registers (up

to 19 of them).

Returns:     All values concatenated in order (string).

Example:     x = "pure"   /* Assign string pure to x. */

ask "Do you want to load the transmittance file?"

/* Ask for type of file. */

if (z == 1)  /* If y is typed...*/

y = ".ras"  /* Assign .ras as the extension. */

else         /* Otherwise... */

y = ".abs"  /* Assign .abs as the extension. */

f = buildstring x y  /* Build a text string that */

/* contains the filename */

/* stored in x and the filename */

/* extension stored in y. */

load f        /* Load the file with the name that */

/* was created. */

###

___calibrate___________________________________________________________

Description: Use to analyze the data in the specified file for

slope, intercept, standard deviation and correlation.

Use this command along with the predictX and predictY

commands to implement routines for quantitative

analysis of single components.  NOTE: You must always

use the calibrate command before the predict commands.

Arguments:   Name of file containing xy pairs (string).  NOTE: This

file must be in ASCII format with one xy pair per line.

The data file for this example (pairs.dat) contains the

following pairs:

1.0  10.0

2.0  22.0

3.0  31.0

4.0  40.0

Example:     int done  /* Declare an integer variable. */

done = 0  /* Set the value of done to 0. */

clrscr    /* Clear the display window. */

print "This program predicts concentration based"

print "on the absorbance amount you enter."

/* Display these messages. */

sleep 5   /* Sleep for five seconds. */

clrscr    /* Clear the screen. */

calibrate "pairs.dat"  /* Calibrate using pairs.dat. */

while (!done)  /* Loop until done is not equal to 0. */

begin     /* Mark top of loop. */

enter a "Enter the absorbance and press Return."

/* Get absorbance value and store in a. */

y = predictY a  /* Predict y using value in a */

/* and store the result in y. */

print "Predicted concentration is at " y

print "when absorbance is " a

/* Print results to screen. */

ask "Do you want to make another prediction?"

if (z == 0)  /* If n is typed... */

done = 1    /* Set done equal to 1. */

end          /* Mark end of loop. */

###

___cascadewindows______________________________________________________

Description: Use to arrange all open data and report windows in a

cascade.

Arguments:   None.

Example:     load pure.abs     /* Load data file. */

peaks             /* Generate peak report. */

load pure2.abs    /* Load data file. */

peaks             /* Generate peak report. */

load pstyrene.abs /* Load data file. */

peaks             /* Generate peak report. */

cascadewindows    /* Arrange all open data and */

/* report windows in a cascade. */

###

___cd__________________________________________________________________

Description: Use to change your current directory.  This command

works the same as the DOS CD command.

Arguments:   Drive and directory path.

Example:     cd \genesis\data\standard  /* Change to the Genesis */

/* directory named */

/* standard. */

###

___clock_______________________________________________________________

Description: Use to report the number of seconds that have elapsed

on the system clock since 00:00:00 Greenwich Mean

Time (GMT), January 1, 1970.  Use the clock command in

combination with the filetime command to determine when

a file was created.  To set a timer to keep track of

seconds, see the timercheck and timerstart commands.

Arguments:   None.

Returns:     The number of seconds.  Also the return value is

stored in the z register.

Example:     /* This program determines the age of a file.  Then, */

/* if the file is more than a week old, the program */

/* calls a user-defined function to acquire another */

/* sample: */

int age      /* Declare integer for the file age. */

int current  /* Declare integer for the current time. */

current = clock  /* Get current time and store */

/* value in current. */

age = filetime "tmp.abs"

/* Get age of tmp.abs file. */

if (( current - age )  > 648000)  /* If too old... */

getnewsamp  /* Call this user-defined operation. */

/* If file is less than 1 week old (one */

/* week is equal to 604800 seconds), do */

/* nothing. */

###

___closeAllWindows_____________________________________________________

Description: Use to close all open data and report windows.  If

global options specify that WinFIRST is to prompt you

before discarding changed data, then the save-changed-

data prompt appears.

Arguments:   None.

Example:     load pure.abs     /* Load data file. */

peaks             /* Generate peak report. */

load pure2.abs    /* Load data file. */

peaks             /* Generate peak report. */

load pstyrene.abs /* Load data file. */

peaks             /* Generate peak report. */

closeallwindows   /* Close all open data and */

/* report windows. */

###

___clrscr______________________________________________________________

Description: Use to clear the text window.

Arguments:   None.

Example:     clrscr  /* Clears all text in the text window. */

###

___compare_____________________________________________________________

Description: Use to correlate sample data with current reference,

or with a newly loaded reference if specified.

Arguments:   (optional) Name of data file to compare (string) with

sample data.

Example:     load "pure1.abs"     /* Load file into memory. */

compare "pure2.abs"  /* Correlate sample with */

/* pure2.abs file. */

###

___compareDir__________________________________________________________

Description: Use to correlate sample data to all data files of the

same type in the specified directory.  CompareDir

searches the directory and displays the file names

and correlation coefficients of the 10 files most

similar to the sample data.

Arguments:   Name of directory containing data to compare (string).

Example:     load "pure1.abs"  /* Load file into memory. */

compare \genesis\data\standard

/* Correlate all files in the */

/* standard directory. */

###

___copy________________________________________________________________

Description: Use to make a copy of a file in another directory,

or to make a copy with a different file name.

Arguments:   One file name (string), and any of the symbols

and switches of the DOS COPY command.

Example:     Copy "pure.abs" "\genesis\backup\pure.abs"

/* Copy the file to the backup directory.*/

###

___cos_________________________________________________________________

Description: Use to compute the cosine of the specified value.

Arguments:   Value (in radians) for which to find cosine (int or

float).

Example:     y = cos 0  /* Place cosine of 0 in register y. */

x = cos z   /* Place cosine of value stored in */

/* register z into register x. */

###

___dataToArray_________________________________________________________

Description: Use to find the array position for a specified

wavenumber.

Arguments:   Wavenumber (float), array to use (variable).

Returns:     Wavenumber associated with position (float).

Example:     load co.abs  /* Load file into memory. */

x = dataToArray 2250.0 sample:data

/* Get array position for */

/* wavenumber 2250.0 and */

/* place value in register x. */

print x       /* Print the value to the screen. */

###

___decrement___________________________________________________________

Description: Use to decrease the value of a register by one.

Arguments:   Register to decrease (register).

Returns:     New value obtained in register (int).

Example:     float array usrarray[5]  /* Declare a floating point */

/* array that can hold 5 */

/* elements. */

x = 5          /* Set value of register x to 5. */

while (x > 0)  /* Loop until x is less than 0. */

begin          /* Mark start of loop. */

enter a "Please enter a wavenumber"

usrarray:data[x] = a  /* Assign the value stored in */

/* register a to the 5th */

/* position in the float array */

/* named 'usrarray'. */

decrement x    /* Decrease value of x by one. */

end            /* Mark end of loop. */

###

___deltaX______________________________________________________________

Description: Use to find the distance (in wavenumbers) between

each data point in the current sample.

Returns:     DeltaX and places value in register z (float).

Arguments:   None.

Example:     load "pure.abs"  /* Load file into memory. */

x = deltax       /* Get delta x and place in x. */

###

___deriv_______________________________________________________________

Description: Use to calculate the first derivative of the sample

data.

Arguments:   None.

Example:     load "pstyrene.abs"  /* Load file into memory. */

deriv  /* Calculate first derivative. */

show   /* Display results. */

###

___dir_________________________________________________________________

Description: Use to list a directory at the ApPro prompt.  This

command works identically to the DOS DIR command.

Arguments:   The same syntax and switches as the DOS DIR command.

Example:     dir "\genesis\data\standard"

/* List all files in the standard directory. */

###

___divide______________________________________________________________

Description:  Use to divide the current sample data with the

data of another file.

Arguments:    The file name of the data by which you want to divide

the sample data in memory.

Example:      load "pure1.abs"       /* Load file into memory. */

divide "pure2.abs"     /* Divide pure1.abs by data */

/* in the pure2.abs file. */

title "pure1 * pure2"  /* Assign title to results. */

show                   /* Display the results. */

###

___dos_________________________________________________________________

Description: Use to execute a DOS internal command or batch file.

DOS internal commands are those that are loaded into

RAM when you first start the computer.  A batch file

contains a batch of DOS commands, which may be internal

or external commands.  For a complete description of

internal and external commands, as well as batch files,

please refer to your DOS manual.

Arguments:   The command or batch file name followed by its

required arguments.  These can be any valid argument

as long as they evaluate to a legal DOS command.

Example 1:   dos "type calib.dat"  /* Display the contents of */

/* the calib.dat file; TYPE */

/* is a DOS command. */

Example 2:  enter x "Enter the file to rename"

/* Ask for file to rename and store in x. */

enter y "Enter the new file name"

/* Ask for new file name and store in y. */

dos "rename" x y  /* Rename the file whose name is */

/* stored in x using the name */

/* stored in y; RENAME is a DOS */

/* command. */

###

___drawbox_____________________________________________________________

Description: Use to draw a box on the displayed data.

Arguments:   The x and y coordinates for a left corner of the box,

and for the right corner that is diagonal to the

left corner you specify.  The x coordinates are in

wavenumbers.  The y coordinates are in absorbance

units (all values are float).

Example:     load "pure.abs"            /* Load file into memory. */

drawbox 3500 0.3 2500 0.2  /* Draw a box at the */

/* specified location. */

###

___drawline____________________________________________________________

Description: Use to draw a line on the displayed data.

Arguments:   The x and y coordinates for both the left and right

ends of the line.  The x coordinates are in

wavenumbers.  The y coordinates are in absorbance

units all values are float).

Example:     load "pure.abs"            /* Load file into memory. */

drawline 3500 0.3 2500 0.2  /* Draw a line at the */

/* specified location. */

###

___drawtext____________________________________________________________

Description: Use to enter text (an annotation) on the displayed

sample data.

Arguments:   The x and y coordinates, with x in wavenumbers and

y as absorbance units (float), followed by the text

to display (string).

Example:     load "pstyrene.abs"  /* Load file into memory. */

show                 /* Display data. */

drawline 1601.02 0.796 2000 1.0    /* Draw line. */

drawtext 2300 1.1 "1601.02 0.796"  /* Draw label. */

###

___dx2matt_____________________________________________________________

Description: Use to a translate a copy of a file in the JCAMP-DX

format to a file in the ATI Mattson FIRST data file

format.  The translated file becomes the current

sample.  Four data types (stored in the JCAMP format)

can be translated: absorbance, transmittance, diffuse

reflectance, and single beam.  The original JCAMP file

that is translated is not removed or otherwise altered

by the dx2matt command.  You can use the translated

data file in the same way you would any data file

created by Genesis.

Arguments:   The directory path and name of JCAMP-DX file (string).

Example:     dx2matt "\JCAMP\data\pstyrene"

/* Translate file and make it the */

/* sample data file. */

###

___enter_______________________________________________________________

Description: Use to query the user for an integer, float, or text

string and place the entered value in the specified

register.

Arguments:   Register, text of question or directions to display

(string).

Example:     enter n "Enter file name to load into memory"

/* Ask user for file name and place in n. */

load n  /* Load file specified by n into memory. */

###

___eof_________________________________________________________________

Description: Use to tell if you are at the end of a file that is

open for input and output.

Returns:     1 if at end of file, and 0 if not (int).  Also, the

return value is stored in the z register.

Arguments:   Number of the file.

Example:     fopen 1 sample.txt  /* Open sample.txt as file 1. */

fread 1 a           /* Read value to register a. */

eof 1     /* Find out if at end of file 1. */

print z   /* Show the contents of the z register. */

###

___erase_______________________________________________________________

Description: Use to erase (delete) a file.  NOTE: Use erase with

care.  Once you erase a file, you cannot retrieve it.

Arguments:   Name of file you want to erase (string).

Example:     erase red.bkg  /* Erase the file. */

###

___errorMsg____________________________________________________________

Description: Use to display an error message or a yes or no

question at the ApPro command line.

Arguments:   Any valid ApPro values: register, variable, or literal.

Returns:     1 if y is typed, 0 if not (int).  Also, the return

value is stored in the z register.

Example:     fopen 1 sample.txt  /* Open sample.txt as file 1. */

fread 1 a           /* Read value to register a. */

eof 1               /* Find out if at end of file 1. */

if (z == 1)         /* If at file end... */

errorMsg "End of file reached...Continue?"

/* Display error message and */

/* ask whether to continue. */

if (z == 0)         /* If y is not typed... */

exit               /* Exit the macro.  Otherwise, */

/* continue the macro. */

###

___escapeHit___________________________________________________________

Description: Use to determine if the Esc key was pressed during

execution of the macro.  This command is useful in

cases where a miscalculation in the code causes an

infinite loop.  If the loop contains code to check for

the Esc key, you can terminate the loop.  NOTE: The

CTRL + BREAK key combination terminates the entire

macro while the escapehit command gives you another

(and sometimes preferable) option.

Arguments:   None.

Return:      Non-zero if Esc is pressed, otherwise 0.

Example:     flushkb        /* Clear the keyboard buffer. */

while (x > 0)  /* Loop while x is greater than 0. */

begin          /* Mark start of loop. */

y = y + x      /* Add value of x to y. */

decrement x    /* Decrease the value of x by 1. */

if (escapehit) /* If Esc key is pressed... */

break         /* Stop the execution of this loop. */

end            /* Mark end of loop. */

###

___execstring__________________________________________________________

Description: Use to execute a text string as ApPro commands.

Arguments:   Text string to execute. (string)

Example:     /* This macro lets you get information for an array: */

string arrname  /* Declare a string to hold */

/* the array name. */

string field    /* Declare a string to hold */

/* the array field. */

int done        /* Declare an integer to control */

/* the loop. */

done = 0        /* Set done equal to 0. */

enter a "Please enter the name of the array"

/* Ask user and store response in a */

arrname  = a    /* Set arrname equal to the text */

/* stored in a. */

/* This prints a list of the valid fields to access: */

print "valid fields are:"

print "firstx, lastx, start, rows, cols, pages, and "

print "data[ array pos ] ([][])"

while (!done)  /* While done is non-zero. */

{              /* Mark start of loop. */

enter a "Enter the array field you want or Q to quit"

if (stringcompare a "Q")  /* If Q is entered... */

done = 1  /* Set done to 1. */

else       /* If the string compare fails... */

{          /* Mark start of else statements. */

field = a  /* Assign text in a to the field */

/* variable. */

execstring (buildstring "print " arrname ":" field)

/* If arrname is "sample" and field is */

/* "firstx", then the string executed is */

/* 'print sample:firstx'. */

}          /* Mark end of else statements. */

}          /* Mark end of loop. */

###

___execute_____________________________________________________________

Description: Use to run and to pass control to any DOS or Windows

executable program.  As control is passed to the

DOS or Windows program, the Genesis software is

terminated (quits running).

Arguments:   Any valid command from the DOS command line (string).

Example:     execute "edlin test.fil"

/* Execute edlin for test.fil. */

###

___exist_______________________________________________________________

Description: Use to determine whether a file exists in the current

directory, or a specified directory.

Arguments:   file name or full path and file name (string).

Return:      1 if file exists, 0 if not (int).  Also, the return

value is stored in the z register.

Example:     exist "\genesis\data\standard\pure.abs"

/* Check to see if file pure.abs exists. */

###

___exit________________________________________________________________

Description: Use for two purposes: 1) to exit a DOS shell (see the

shell command) and return to Genesis and ApPro, and

2) to exit an ApPro macro or ApPro command line.

Arguments:   None.

Example:     shell  /* Create a temporary copy of DOS.  The DOS */

/* prompt appears. */

exit   /* Close the DOS copy (shell) and return to */

/* Genesis or ApPro. */

###

___exp_________________________________________________________________

Description: Use to calculate e to the x for a given value, or

for the sample data if no value is given.

Arguments:   (optional) Value to use to compute e to the x (float).

Returns:     Float if a value is specified.

Example 1:   load "pstyrene.abs"  /* Load file into memory. */

exp                  /* Calculate e to the x for */

/* each data point in the */

/* sample data. */

show                 /* Show results. */

Example 2:   print (exp 2.5)      /* Calculate e to the 2.5. */

###

___expand______________________________________________________________

Description: Use to expand the display of a region of the

sample data.  NOTE: Use the autoscale command to

restored the unexpanded data display.

Arguments:   Two wavenumbers and two y axis values, as follows:

maximum wavenumber, minimum wavenumber, maximum y

value, minimum y value (float).

Example:     load "pstyrene.abs"       /* Load file into memory. */

expand 3500 2500 2.5 0.0  /* Expand region. */

show                      /* Show expanded display. */

autoscale                 /* Show as unexpanded. */

###

___expandX_____________________________________________________________

Description: Use to show an enlarged or expanded region of the

SAMPLE data along the x axis.  NOTE: Use the

autoscale command to restore the unexpanded data

display.

Arguments:   Two wavenumbers, as follows: maximum wavenumber,

minimum wavenumber (float).

Example:     load "pstyrene.abs"  /* Load file into memory. */

expandx 3500 2500    /* Expand display in x region. */

show                 /* Show expanded display. */

autoscale            /* Show unexpanded display. */

###

___expandY_____________________________________________________________

Description: Use to show an expanded region of the sample data

along the y axis.  NOTE: Use the autoscale command to

restores the unexpanded data display.

Arguments:   Two y axis values, as follows: maximum y value, min y

value (float).

Example:     load "pstyrene.abs"  /* Load file into memory. */

expandy 2.0 0.0      /* Expand display in y region. */

show                 /* Show expanded display. */

autoscale            /* Show unexpanded display. */

###

___fclose______________________________________________________________

Description: Use to close a data file that is open for read and

write.

Arguments:   File number (int).

Example:     fopen 1 "file1.dat"  /* Open file1.dat for read and */

/* write. */

fread 1 a            /* Read first value in file 1*/

/* and store in register a. */

fclose 1             /* Close file number 1. */

###

___fft_________________________________________________________________

Description: Fft is an object that contains properties and methods

you can set and execute to perform fast fourier

transform processing of interferogram data.

NOTE: Enclose all strings entered as arguments within

double quotation marks.  For example:

load test.igm    /* Load interferogram file. */

fft:setTodata    /* As a needed first step, set */

/* all current fft process */

/* properties to match those used */

/* to process active sample data. */

/* This step assures properties, */

/* like ndata, symmetry, etc. */

/* have valid settings.  */

fft:ftApod = "triangle"  /* Set properties as */

/* needed. */

fft:process      /* Process interferogram data. */

Properties:  Properties for the fft object are:

fstart         Use to specify a wavenumber for the start of the

frequency range of the processed data file.

Arguments:     Enter a value (int or string) appropriate for the

spectral range that the input data represents:

For NIR, the default starting value is 2000

wavenumbers.  Enter a value from 1 to 15,798.

For MIR, the default starting value is 400

wavenumbers.  Enter a value from 1 to 7,899.

For FIR, the default starting value is 50

wavenumbers.  Enter a value from 1 to 1,974.

For VIS, the default starting value is 15,798

wavenumbers.  Enter a value from 15,798 to 31,596.

For UV, the default starting value is 31,596.

Enter a value from 31,596 to 47,393.

Syntax:        fft:fstart = 400

fend           Use to specify a wavenumber for the end of the

frequency range of the processed data file.

Arguments:     Enter a value (int or string) appropriate for the

spectral range that the input data represents:

For NIR, the default starting value is 2000

wavenumbers.  Enter a value from 1 to 15,798.

For MIR, the default starting value is 400

wavenumbers.  Enter a value from 1 to 7,899.

For FIR, the default starting value is 50

wavenumbers.  Enter a value from 1 to 1,974.

For VIS, the default starting value is 15,798

wavenumbers.  Enter a value from 15,798 to 31,596.

For UV, the default starting value is 31,596.

Enter a value from 31,596 to 47,393.

Syntax:        fft:fend = 4000

ndata:         Sets the number of data points to use in the FFT

calculation (the number of interferogram data

points to transform).  This value (along with

lasersamp and FFT Symmetry) determines the

resulting spectrum's optical resolution.  Notice

that ndata can be different than nfft.

NOTE: Set the ndata property only if you are

knowledgeable in the area of FFT processing.

Arguments:     Any value (int or string).

Syntax:        fft:ndata = 1024

nfft:          Use to specify the number of Fast Fourier

Transform points, that is, the total number of

data points to transform.  FFT Points determines

digital resolution, defined as the distance

between output data points.

Arguments:     A value (int or string) to specify the number of

phase FFT points.  This number must be a power of

2, no smaller than 1024 and no greater than 512K,

or 524288.

Syntax:        fft:nfft = 1024

ftsymmetry:    Use to specify FFT symmetry as either double or

single.

Arguments:     The name (string) for the type of symmetry you

want to use: single, or double.

Syntax:        fft:ftsymmetry = "double"

ftapod:        Use to specify the apodization method to use.

Arguments:     The name for the type of apodization method you

want to use: boxcar, triangle, Happ Genzel,

Bessel, Welch, Beer Norton Weak, Gaussian, Beer

Norton Medium, Beer Norton Strong, Hanning, Cosine

Squared, and Cosine 4th (string).

Syntax:        fft:ftapod = "triangle"

PhaseNfft:     Use to specify the number of phase FFT points.

(For example, if overshoot, or ringing appears at

the edge of a strong, wide band, larger values for

PhaseNfft, and also nPhaseData may be needed.)

Arguments:     A value (int or string) to specify the number of

phase FFT points.  This number must be a power of

2, no less than 64 and no more than 512K, or

524288.

Syntax:        fft:PhaseNfft = 1024

nPhaseData:    Use to specify the number of phase data points.

Arguments:     A value (int or string) to specify the number of

phase data points.

Syntax:        fft:nPhaseData = 300

ftPhaseMethod: Use to specify the phase correction method.

Arguments:     The name for the type of phase correction method

you want to use: Mertz, Stored, Power, and None

(string).

Syntax:        fft:ftPhaseMethod = "power"

ftPhaseApod:   Use to specify the phase apodization method.

Arguments:     The name for the type of phase apodization you

want to use: boxcar, triangle, Happ Genzel,

Bessel, Welch, Beer Norton Weak, Gaussian, Beer

Norton Medium, Beer Norton Strong, Hanning, Cosine

Squared, and Cosine 4th (string).

Syntax:        fft:ftPhaseApod = "Happ Genzel"

ftPhaseType:   Use to specify the phase type used to perform

the FFT.

Arguments:     The name for the type of phase type you want to

use: imaginary or real (string).

Syntax:        fft:ftPhaseType = "imaginary"

ftOutput:      Use to specify the output form of the processed

data.

Arguments:     The name of the output form you want the FFT

processing to create: single beam, transmittance,

absorbance, and diffuse reflectance (string).

Syntax:        fft:ftOutput = "single beam"

refSource:     Use to specify the source for the background data

file to use in processing the interferogram data.

Arguments:     The name for the background data source you want

to use in FFT processing: Current Background, Disk

File (file open dialog prompts you to select

file), or Current Window (string).

Syntax:        fft:refSource = "current background"

Methods:     Methods for fft are:

process:       Use to perform the FFT.

Arguments:     None.

Syntax:        fft:process

setToData:     Use to set current fft properties to match those

used to process the current sample data file (as

specified in data file header).  Modify individual

property settings as needed.

Arguments:     None.

Syntax:        fft:setToData

makeBackground: Use to make the processed data the current

background.

Arguments:     None.

Syntax:        fft:makeBackground

readMethod:    Use to load a math method (configuration).  The

fft property settings specified by the math method

are put in place as the current settings.

Arguments:     The name of the method file to load (string).  If

you do not specify a file name, the Load Math Setup

dialog appears.

Syntax:        fft:readMethod = "forfft"

saveMethod:    Use to save a math method (configuration).  The

current fft property settings are saved as part of

the method.

Arguments:     The name of the method file to save (string).  If

you do not specify a file name, the Save Math

Setup dialog appears.

Syntax:        fft:savemethod

###

___filePos_____________________________________________________________

Description: Use to determine the current position of a text or

binary file that is open for read and write.  The file

position is given in number of bytes from the beginning

of the file.

Arguments:   File number (int).

Returns:     The position, in bytes.  Also, the return value is

stored in the z register (int).

Example:     fopen 1 "file1.dat"  /* Open file 1 for read and */

/* write. */

print (filepos 1)    /* Print file position for */

/* file 1. */

###

___filetime____________________________________________________________

Description: Use to determine the time a file was created.  Use

the filetime command in combination with the clock

command to determine when a file was created.  To set

a timer to keep track of seconds, use the timercheck

and timerstart commands.

Arguments:   The name of the file whose creation time you want to

determine. (string)

Returns:     The time, in seconds.  Also, the return value is stored

in the z register.

Example:     /* This program determines the age of a file.  Then, */

/* if the file is more than a week old, the program */

/* calls a user-defined function to acquire another */

/* sample: */

int age          /* Declare integer for file age. */

int current      /* Declare integer for current */

/* time. */

current = clock  /* Get the current time and store */

/* the value in current. */

age = filetime "tmp.abs"  /* Get the creation time */

/* of the tmp.abs file. */

if (( current - age )  > 648000)  /* If too old... */

getnewsamp      /* Call this user-defined function. */

/* If file is less than 1 week old */

/* (604800 seconds = 1 week), do */

/* nothing. */

###

___fillArea____________________________________________________________

Description: Use to fill a specified wavenumber range to a

calculated baseline between two wavenumbers.

Arguments:   Two wavenumbers (float).

Example:     load "pstyrene.abs"    /* Load file into memory. */

fillarea 3000 2500     /* Fill wavenumber region to */

/* calculated baseline. */

show                   /* Show the results. */

###

___findFirstFile_______________________________________________________

Description: Use to find the first occurrence of a set of files with

similar names within the current directory.

Arguments:   Any valid DOS filename with optional wildcards, ? and

* (string).

Returns:     The name of the first file found, or an empty string if

nothing is found.

Example:     /* This prints the names of all files in the current */

/* directory that have .mac as a filename extension: */

s = findfirstfile "*.mac"  /* Put the first filename */

/* found into register s. */

a = stringlength s         /* Put length of string */

/* in s into register a. */

while ( a > 0 )     /* Loop until a is 0 length. */

{                   /* Mark start of loop. */

print s             /* Print file name to screen. */

s = findnextfile    /* Get next occurrence of *.mac. */

a = stringlength s  /* Put string length into a. */

}                   /* Mark end of loop. */

###

___findNextFile________________________________________________________

Description: Use to find the next occurrence of the set of files

specified by the findfirstfile command.

Arguments:   None.

Returns:     The name of the file found, or an empty string if

nothing is found.

Example:     /* This prints the names of all files within the */

/* current directory that have .mac as a filename */

/* extension: */

s = findfirstfile "*.mac"  /* Put the first filename */

/* found into register s. */

a = stringlength s         /* Put length of string */

/* in s into register a. */

while ( a > 0 )     /* Loop until a is 0 length. */

{                   /* Mark start of loop. */

print s             /* Print file name to screen. */

s = findnextfile    /* Get next occurrence of *.mac. */

a = stringlength s  /* Put string length into a. */

}                   /* Mark end of loop. */

###

___finput______________________________________________________________

Description: Use to read one line of text from a text file.

Arguments:   File number of an opened file (int), register in which

to store the input.

Example:     fopen 1 "testfile.dat"  /* Open testfile.dat file. */

finput 1 a              /* Read first line in file. */

finput 1 b              /* Read next line in file. */

fclose 1                /* Close the file. */

print "line 1  :  " a   /* Print the contents of */

/* register a to the screen. */

print " line2  :  " b   /* Print the contents of */

/* register b to the screen. */

###

___fkeys_______________________________________________________________

Description: Use to display a row of function keys along the bottom

of the Genesis screen.  You can use the function keys

created exactly as you use other Genesis function keys

while the macro is running.  You can create up to eight

function keys at one time.  Use the getfkey command to

retrieve input from the function keys.

Arguments:   List of the labels you want to assign to the function

keys (string).  Each label name must be followed by a

semi-colon (;) within the label list.

Example:     int choice  /* Declare integer variable to hold key */

/* selection. */

int done    /* Declare integer variable for loop */

/* control. */

done = 0    /* Set done equal to 0. */

fkeys "Sample; Bkg; Plot; Done"

/* Define function keys. */

while (done == 0)  /* Loop until done is non-zero. */

{                  /* Mark start of loop. */

choice = getFkey "Press a function key..."

/* Wait for user to press a function */

/* key.  After a key is pressed, the */

/* key number is stored in choice. */

if (choice == 1)   /* If first Fkey pressed... */

collectsample     /* Call this user function. */

else if (choice == 2)  /* If second Fkey pressed... */

collectbkg            /* Call this user function. */

else if (choice == 3)  /* If third Fkey pressed... */

plotcurrentdata       /* Call this user function. */

else if (choice == 4)  /* If fourth Fkey pressed... */

done = 1              /* Set done to 1 to terminate */

/* the loop. */

}                      /* Mark end of while loop. */

###

___flushKb_____________________________________________________________

Description: Use to clear the keyboard buffer when running a macro.

For example, use this command when using the escapeHit

command.  (After using flushKb, you are sure the

keyboard buffer is empty.  Then when the Esc key is

pressed, it is the first key in the buffer.)

Arguments:   None.

Example:     flushkb         /* Clear the keyboard buffer. */

while (x > 0)   /* Loop while x is greater than 0. */

begin           /* Mark start of loop. */

y = y + x       /* Add value of x to y. */

decrement x     /* Decrease value of x by 1. */

if (escapehit)  /* If Esc key pressed... */

break;         /* Stop executing this loop. */

end             /* Mark end of loop. */

###

___fopen_______________________________________________________________

Description: Use to open a file for reading and writing.  Up to 16

files can be open at one time.

Arguments:   File number, from 1 to 16 (int), file name (string).

Example:     fopen 1 "file1.dat"  /* Open file1.dat for read */

/* and write. */

fread 1 a            /* Read first value in file 1 */

/* into register a. */

fclose 1             /* Close file number 1. */

###

___fprint______________________________________________________________

Description: Use to print text to an open (text) file.

Arguments:   File number, from 1 to 16 (int), text to print to file

(up to 19 literal values, variables or registers).

Example:     fopen 1 "file1.dat"  /* Open file1.dat for read */

/* and write. */

a = "file1.dat"      /* Set a equal to "file.dat". */

fprint 1  "Filename :  " a  /* Print to the file. */

fclose 1             /* Close file number 1. */

###

___fread_______________________________________________________________

Description: Use to read binary data from an open user file.  One

value is read at a time.

Arguments:   File number, from 1 to 16 (int), file name (string).

Example:     fopen 1 "file1.dat"  /* Open file1.dat for read */

/* and write. */

fread 1 a            /* Read first value in file 1 */

/* into register a. */

fclose 1             /* Close file number 1. */

###

___fseek_______________________________________________________________

Description: Use to move to a specific position in a text or binary

file that is open for read and write.

Arguments:   File number (int), number of bytes into the file to

move (int).

Returns:     1 if successful, 0 if not (int).

Example:     fopen 1 "file.dat"  /* Open file.dat for read and */

/* write. */

fseek 1  32         /* Seek 32 bytes into the file. */

fread 1 a           /* Read the next value in file. */

fclose 1            /* Close the file. */

###

___fwrite______________________________________________________________

Description: Use to write values to an open (binary) file.

Arguments:   File number, from 1 to 16 (int ), value to print to

file.

Example:     fopen 1 "file1.dat"  /* Open file1.dat for read and */

/* write. */

fwrite 1  2500.00    /* Write to the file. */

fclose 1             /* Close file number 1. */

###

___getFirstDrive_______________________________________________________

Description: Use to return a string that contains the drive on which

WinFIRST (or Genesis) is installed.  The returned

string takes the form: "c:", that is, a colon is

appended to the drive identifier.  This command is

useful for building directory names that contain data

for a macro, so that you can install the macro on

different drives and different computers.

Arguments:   None.

Example 1:   cd (getFirstDrive)  /* Change to the WinFIRST drive. */

Example 2:   string myDir

s = getFirstDrive

myDir = s + "\first\data\mydata"

cd myDir

###

___getFkey_____________________________________________________________

Description: Use to display a message and wait for the user to press

a function key.  Use this command with the fkeys

function for customized function keys.

Arguments:   Text to display while waiting for user to press a key

(string).

Returns:     The number of the key the user pressed (int).

Example:     int choice  /* Declare integer variable to hold */

/* key selection. */

int done    /* Declare integer variable for loop */

/* control. */

done = 0    /* Set done equal to 0. */

fkeys "Sample; Bkg; Plot; Done"

/* Define function keys.*/

while (done == 0)  /* Loop until done is non-zero. */

{                  /* Mark start of loop. */

choice = getFkey "Press a function key..."

/* Wait for user to press function key. */

/* The key user presses is stored in */

/* choice. */

if (choice == 1)   /* If first Fkey pressed... */

collectSample     /* Call this user function. */

else if (choice == 2)  /* If second FKey pressed... */

collectBkg            /* Call this user function. */

else if (choice == 3)  /* If third Fkey pressed... */

plotCurrentData       /* Call this user function. */

else if (choice == 4)  /* If fourth Fkey pressed... */

done = 1   /* Set done to 1 to terminate loop. */

}           /* Mark end of the while loop. */

###

___getStringElement____________________________________________________

Description: Use to retrieve an individual character within a

string.

Arguments:   String from which to get the character (string),

position of the character you want to retrieve (int).

Returns:     The ASCII value of the character.  This value is an

integer.  To get the actual character, assign the

return value to a char variable.

Example 1:   x = getstringelement "This is a string" 4

/* The ASCII value of the 's' */

/* character (located in */

/* position 4) is stored in x. */

Example 2:   char ch             /* Declare a character variable. */

ch = getstringelement "This is a string" 4

/* The actual character 's' is */

/* stored in ch. */

###

___help________________________________________________________________

Description: Use to display information about a specific ApPro

command, or to display a list of all commands for

which online help available.  With this list displayed,

select a listed command to display help information

for that command.

Arguments:   (optional)  Name of command for which you want to

display help information (string).

Example 1:   help getStringElement  /* Information for */

/* getstringelement is */

/* displayed. */

Example 2:   help  /* A list of available commands is displayed. */

/* Select a listed command to display help */

/* for that command. */

###

___include_____________________________________________________________

Description: Use to execute an ApPro macro.  You can use the

include command from the ApPro command line or from

within a macro.

Arguments:   The name of the program (string).

Example:     Create these macro files, "mac1.mac" and "mac2.mac",

as follows:

The mac1.mac file contains:

print "In mac1.mac... calling mac2.mac"

include "mac2.mac"        /* Include mac2.mac file. */

print "Back in mac1.mac"  /* This line is executed */

/* when mac2.mac is done */

/* executing. */

The mac2.mac file contains:

print "In mac2.mac..."

print "At last line.  Returning to mac1.mac..."

To run these macros:

At the ApPro command line type:  run "mac1.mac"

You can also execute this macro by using the load

Fkey in the scan administration macros set of Fkeys.

###

___increment___________________________________________________________

Description: Use to increase the value of a register by 1.

Arguments:   Register to increase (register).

Returns:     New value obtained in register (int).

Example:     float array usrarray[5]  /* Declare a floating point */

/* array that can hold up */

/* to 5 elements. */

x = 1          /* Set value of register x to 1. */

while (x < 6)  /* Loop until x is greater than 6. */

begin          /* Mark start of loop. */

enter a "Please enter a wavenumber."

usrarray:data[x] = a  /* Assign the value stored in */

/* register a to the xth */

/* position in the float array */

/* 'usrarray'. */

increment x    /* Increment value of x by 1. */

end           /* Mark end of loop. */

###

___info________________________________________________________________

Description: Use to print the data information report for the

current sample.

Arguments:   None.

Example:     info  /* Print the sample data information report. */

###

___inp_________________________________________________________________

Description: Use to read a byte from the specified input port.

Arguments:   The input port (an integer ranging from 0 to 65535).

Returns:     The byte read from the port.

Example:     byte portinput          /* Declare a byte to hold */

/*port input. */

portinput = (inp 0x61)  /* Read in a byte from port */

/* 0x61 (hex). */

###

___inpw________________________________________________________________

Description: Use to read a word from the specified input port.

Arguments:   The input port (an integer ranging from 0 - 65535)

Returns:     The word read from the port.

Example:     int portinput             /* Declare an integer to */

/* hold port input. */

portinput = (inpw 0x61)]  /* Read in a word from */

/* port 0x61 (hex). */

###

___integrate___________________________________________________________

Description: Use to find the area of a data region defined by the

specified wavenumbers and a calculated baseline that

connects the two wavenumbers.  The integrate command

fills in the data area that is integrated, and prints

an integration report to the text window.

Arguments:   Two wavenumbers (float).

Returns:     The calculated value (float).  Also, the return value

is stored in the z register.

Example:     load "pstyrene.abs"   /* Load file into memory. */

abc                   /* Correct baseline */

/* automatically. */

integrate  3000 2500  /* Find area between */

/* 3000 and 2500 wavenumbers. */

###

___interpolate_________________________________________________________

Description: Use to estimate a different digital resolution.

Arguments:   The new resolution, from 0.1 to 64.0 wavenumbers

(float).  NOTE: If the value is out of range, no

estimate is made.

Example:     load "pstyrene.abs"  /* Load the file into memory. */

interpolate 4.0      /* Change data resolution to */

/* 4.0 wavenumbers. */

show                 /* Display the resulting data. */

###

___invert______________________________________________________________

Description: Use to invert the sample data.

Arguments:   None.

Example:     load "pstyrene.abs"  /* Load the file into memory. */

invert               /* Invert the sample data. */

show                 /* Display the resulting data. */

###

___irChart_____________________________________________________________

Description: Use to display the WinFIRST Correlation Chart workset.

Arguments:   (optional)  The name of the correlation chart to use.

Example:     load "pstyrene.abs"  /* Load file into memory. */

irChart charta       /* Display the correlation */

/* chart workset and load */

/* the charta.cht */

/* correlation chart. */

###

___isNumber____________________________________________________________

Description: Use to determine whether the specified value is a

valid number.

Arguments:   Any literal value, variable, or register.

Example:     fopen 1 "testfile.dat"  /* Open testfile.dat file. */

finput 1 a              /* Read first line in file. */

fclose 1                /* Close the file. */

if (isnumber a)  /* If text in a is valid number... */

print "The value stored in a is a valid number."

/* Display the message. */

###

___launch______________________________________________________________

Description: Use to execute a Windows or DOS application

concurrently with Genesis.  (Genesis does not need to

wait for the application to terminate.)  If the

application  is unsuccessful, an error message is

displayed.

Arguments:   The command that starts the application (string).

Usually this consists of a program name and 0 or more

command line arguments.

Returns:     1 if successful, 0 if not (int).

Example:     launch "pbrush.exe"

###

___library_____________________________________________________________

Description:  Library is an object with prperties and methods you

can use to create and modify user libraries.

NOTE: The library object works with user libraries only.

See examples for using the library object at the end of

this description.

Properties:   Properties for the library object are:

name:          The name of the user library to use, that is, the

name of library that is to be the current library.

Methods:     Methods for the library object are:

numSpectra:    Use to return the number of spectra in the

current library.  Retruns 0 if the library does

not exist or if no spectra are in the library.

Arguments:     None.

Syntax:        library:numspectra

add:           Use to add the current sample data to the current

library.  The data is always added to the end of

the library.  Returns 0 if successful.  Returns 1

for failure.

Arguments:     None.

Syntax:        library:add

retrieve:      Use to retrieve the spectrum of the specified

index from the current library, and place that

spectrum in a new data window.  Returns 0 if

successful.  Returns 1 for failure.

Arguments:     The library index for the spectrum to retrieve.

Syntax:        library:retrive index

Example 1:     /* This macro plots the entire contents of a */

/* library.  It assumes a plot method named */

/* "libplot" has already been created using */

/* the plot composer. */

library:name = "icon-1"

a = library:numSpectra

if (ask "Prints " a " pages.  Press N to cancel.")

{

for i = 1 to a step 1

{

if (library:retrieve i)

{

plot "libplot"

/* libplot is a plot method. */

sample:clear  /* So 500 data windows are */

/* not opened. */

}

else

{

print "ERROR getting " i " from icon-1"

break;

}

}

}

Example 2:     /* This macro puts the titles of all spectra */

/* in a library into a report window.  Note the */

/* use of the setStringElement command to get a */

/* tab stop into a string.  The tab character */

/* ASCII 9. */

string tab

tab = " "

tab = setStringElement tab 1 9

/* Above changes the space to a tab. */

library:name = "icon-1"

a = library:numSpectra

report:new

for i = 1 to a step 1

{

if (library:retrieve i)

{

s = buildString i tab sample:name tab sample:title

print s

sample:clear

}

else

{

print "ERROR getting " i " from icon-1"

break;

}

}

###

___lineout_____________________________________________________________

Description:  Use to replace the data between two specified

wavenumbers with a straight line.

Arguments:    The starting and ending wavenumbers of the data you

want to line out.

Example:      load "pstyrene.abs"  /* Load file into memory. */

show                 /* Display the spectrum. */

lineout 4000 3200    /* Replace the data between */

/* 4000 and 3200 wavenumbers */

/* with a straight line. */

show                 /* Display the results. */

###

___linreg (Linear Regression)__________________________________________

Description: Use to calculate the slope, intercept, and standard

deviation of peaks in the current sample.  These

values are printed to the Genesis report window.

Arguments:   None.

Example:     load "pstyrene.abs"  /* Load file into memory. */

autoscale            /* Display the data autoscaled. */

abc                  /* Correct the baseline */

/* automatically. */

linreg               /* Calculate the linear */

/* regression of the peaks. */

###

___ln__________________________________________________________________

Description: Use to calculate the natural logarithm of the given

value, or of the current sample if no value is given.

Arguments:   (optional) Value for which you want to compute the

natural log (float).

Returns:     (float), if value specified.

Example 1:   load "pstyrene.abs"  /* Load file into memory. */

ln                   /* Calculate ln of each */

/* data point. */

show                 /* Display the resulting data. */

Example 2:   print (exp 2.5)      /* Calculate ln of 2.5. */

###

___load________________________________________________________________

Description: Use to open (or load) a data file into memory

and display the data in the data display area.

Arguments:   The name of the file you want to load.  If the file

is not in the current directory, ApPro searches all

other Genesis data directories for the file.  If the

file is not in a Genesis directory, you must specify

the complete file path (including drive designation).

Example:     load "c:\genesis\data\standard\pure1.abs"

/* Load the file into memory*/

/* and display it in the data */

/* display area. */

###

___loadRef_____________________________________________________________

Description: Use to open (or load) one or multiple data files into

the active data window as a reference data files.

Arguments:   (optional)  The name of the file you want to load.  If

the file is not in the current directory, ApPro

searches all other WinFIRST data directories for the

file.  If the file is not in a WinFIRST directory, you

must specify the complete file path (including drive

designation).  If you provide no argument, a multiple-

file load dialog appears.

Example:     loadRef "c:\first\data\galaxy\pure1.abs"

/* Load the file into memory and display */

/* it as reference data in the active */

/* data window. */

###

___log_________________________________________________________________

Description: Use to calculate the base 10 logarithm of the given

value, or of the current sample if no value is given.

Arguments:  (optional) Value for which you want to compute the base

10 log (float).

Returns:    (float), if value specified.

Example 1:  load "pstyrene.abs"    /* Load file into memory. */

log                    /* Calculate log of each */

/* sample file data point. */

show                   /* Display resulting data. */

Example 2:  print (log 2.5)        /* Calculate the log of 2.5. */

###

___matt2dx_____________________________________________________________

Description: Use to convert a copy of the current sample data

to the JCAMP file format.  You can convert four data

types: absorbance, transmittance, diffuse reflectance,

and single beam.  The original data file is not

affected.

Arguments:   (optional)  The name of the converted file (string).

if you do not specify a name for the converted file,

the sample file filename is used with .dx assigned

as the filename extension.

Returns:     1 if successful, 0 if not (int).  Also, the return

value is stored in the z register.

Example:     load "pstyrene.abs"  /* Load file into memory. */

matt2dx              /* Convert file to the JCAMP */

/* format and use pstyrene.dx */

/* as the file name. */

matt2dx "pstyren2"   /* Convert file to the JCAMP */

/* format and use pstyren2.dx */

/* as the file name. */

###

___matt2sadtler________________________________________________________

Description: Use to convert the current sample data to the Sadtler

data file format (.irf).

Arguments:   (optional)  The name of the converted file (string).

If you do not specify a name for the converted file,

the sample file filename is used with .irf as the

filename extension.

Returns:     1 if successful, 0 if not (int).  The return value is

stored in the z register.

Example:     load "pstyrene.abs"  /* Load the file into memory */

matt2sadtler         /* Convert file to the Sadtler */

/* format and use pstyrene.irf */

/* as the file name. */

matt2sadtler "pstyr2"  /* Convert file to the */

/* Sadtler format and use */

/* pstyr2.irf as a file name. */

###

___matt2spc____________________________________________________________

Description: Use to convert the current sample data to the Spectra

Calc file format.

Arguments:  (optional)  The name of the converted file (string).  If

you do not specify a name for the converted file, the

sample file filename is used with .spc assigned as the

filename extension.

Returns:    1 if successful, 0 if not (int).  Also, the return value

is stored in the z register.

Example:    load "pstyrene.abs"    /* Load the file into memory */

matt2spc     /* Convert file to the Spectra Calc */

/* format and use pstyrene.spc as the */

/* file name. */

matt2dx "pstyren2"

/* Convert file to the Spectra Calc */

/* format and use pstyren2.spc as the */

/* as the file name. */

###

___matt2xy_____________________________________________________________

Description: Use to write the current sample data to a file as a

table of x and y values.

Arguments:   (optional)  The name of the converted file (string).

If you do not specify a name for the converted file,

the sample file filename is used with .tbl assigned as

the filename extension.

Returns:     1 if successful, 0 if not (int).  Also, the return

value is stored in the z register.

Example:     load "pstyrene.abs"  /* Load file into memory. */

matt2xy              /* Convert file to an xy table */

/* and use pstyrene.tbl as the */

/* file name. */

matt2xy "pstyren2"   /* Convert file to an xy table */

/* and use pstyren2.tbl as the */

/* file name. */

###

___max_________________________________________________________________

Description:  Use to find the maximum y axis value for the current

sample data.

Arguments:    None.

Returns:      The y maximum (float).  Also, the return value is

stored in the z register.

Example:      load "pstyrene.abs"  /* Load file into memory. */

max                  /* Retrieve largest y value. */

###

___maxX________________________________________________________________

Description:  Use to find the maximum x axis value for the current

sample data.

Arguments:    None.

Returns:      The x maximum (float).  Also, the return value is

stored in the z register.

Example:      load "pstyrene.abs"  /* Load file into memory. */

maxx                  /* Retrieve largest x value. */

###

___menu________________________________________________________________

Description: Use to place a menu box on the screen.  Once the menu

box is displayed, you can select any of the numbered

items by clicking on it, pressing the number associated

with it, or placing the highlight over it and pressing

carriage return.

Arguments:   The list of the text items to be placed in the menu

(string).  The list is one text string that contains

each menu text item separated by a semi-colon.  For

example: "Sample;Background;Plot;Quit"

Returns:     The number of the menu item that is selected (int).  If

the Escape key is pressed, or if the mouse is clicked

outside the menu box, a -1 is returned (the menu has

been terminated without a choice).  Also, the return

value is stored in the z register.

Example:     int choice  /* Declare integer variable to hold */

/* item selection. */

int done    /* Declare integer variable for loop */

/* control. */

done = 0    /* Set done equal to 0. */

while (done == 0)  /* Loop until done is non-zero. */

{           /* Mark start of loop. */

choice = menu "Sample;Bkg;Plot;Quit"

/* Wait for menu item to be selected, */

/* and after item is selected store the */

/* item number in choice. */

if (choice == 1)  /* If first item selected... */

collectSample    /* Call this user function. */

else if (choice == 2)  /* If second item selected... */

collectBkg            /* Call this user function. */

else if (choice == 3)  /* If third item selected... */

plotCurrentData       /* Call this user function. */

else if (choice == 4)  /* If fourth item selected... */

done = 1    /* Set done to 1 to terminate the loop. */

else if (choice == -1)  /* If menu is terminated... */

done = 1    /* Set done to 1 to terminate the loop. */

}            /* Mark the end of the while loop. */

###

___md__________________________________________________________________

Description: Use to create a new DOS directory.

Arguments:   The name of the directory to create.

Example:     md c:\first\data\mine  /* Create the directory. */

/* MD is a DOS command. */

md junk

###

___min_________________________________________________________________

Description: Use to find the minimum y axis value for the current

sample.

Arguments:   None.

Returns:     The y minimum (float).  Also, the return value is

stored in the z register.

Example:     load "pstyrene.abs"  /* Load file into memory. */

min                  /* Retrieve lowest y value. */

###

___minX________________________________________________________________

Description: Use to find the minimum x axis value for the current

sample.

Arguments:   None.

Returns:     The x minimum (float).  Also, the return value is

stored in the z register.

Example:     load "pstyrene.abs"  /* Load file into memory. */

minx                 /* Retrieve lowest x value. */

###

___multiply____________________________________________________________

Description: Use to multiply each data point for the current sample

with the data of another file.

Arguments:   The file name of the data by which you want to multiply

the sample data in memory.

Example:     load "pure1.abs"       /* Load file into memory. */

multiply "pure2.abs"   /* Multiply pure1.abs by the */

/* data in pure2.abs file. */

title "pure1 * pure2"  /* Title the results. */

show                   /* Display the results. */

###

___neta________________________________________________________________

Description: Use to calculate the net absorbance for the specified

wavenumber to the specified baseline.

Arguments:   Three wavenumbers (float).  The first two wavenumbers

define the baseline.  The third wavenumber is the data

point for which you want to calculate net absorbance.

Returns:     The net absorbance (float).  Also, the return value is

stored in the z register.

Example:     load "pstyrene.abs"  /* Load file into memory. */

neta 1500.0 1600.0 1550

/* Calculate net absorbance */

/* from the data point at */

/* 1550 wavenumbers to the */

/* baseline defined by 1500 */

/* and 1600 wavenumbers. */

###

___normalize___________________________________________________________

Description: Use to move the lowest data point to zero and move all

other data points by the same amount.

Arguments:   None.

Example:     load "pure2.abs"  /* Load file into memory. */

autoscale         /* Display auto scaled data. */

normalize         /* Normalize the data. */

show              /* Display the results. */

###

___offset______________________________________________________________

Description: Use to offset the data along the y axis by a specified

value.

Arguments:   The offset value (float).

Example:     load "pure2.abs"  /* Load file into memory. */

autoscale         /* Display autoscaled data. */

offset 2.0        /* Offset by 2.0 absorbance units. */

show              /* Display the results. */

###

___outp________________________________________________________________

Description: Use to write a byte to the specified output port.

Arguments:   The port to which to send the byte (integer between

0 and 65535), the value to write to the port (integer

between 0 and 255).

Return:      The value that is written to the port (byte).

WARNING:     This command is potentially very dangerous!  Before

you write to a port, make sure you know what the port

controls and that you understand the consequences of

your actions.

Example:     outp 0x61  182  /* Write 182 to port 0x61 (hex). */

###

___outpw_______________________________________________________________

Description: Use to write a word to the specified output port.

Arguments:   The port to which to send the word (integer between 0

and 65535), the value to write to the port (integer

between 0 and 65535).

Return:      The value that is written to the port (byte).

WARNING:     This command is potentially very dangerous!  Before

you write to a port, make sure you know what the port

controls and that you understand the consequences of

your actions.

Example:     outpw 0x61  1232  /* Write 1232 to port 0x61 (hex). */

###

___overlay_____________________________________________________________

Description: Use to overlay the sample and reference spectra.  Also

use to load a reference spectrum to overlay with the

sample (and other loaded references).

Arguments:   (optional)  The name of the reference data file to load

and overlay.  If you specify no file name, the current

references and sample are overlaid, one on top of

another.

Example:     load "mix2.abs"    /* Load file into memory. */

overlay "mix.abs"  /* Overlay mix.abs with mis2.bas */

/* and display the results. */

###

___p2pNoise____________________________________________________________

Description: Use to calculate peak-to-peak noise for the current

sample spectrum within a wavenumber range you specify.

If verbose is set to "on" the calculated value is

automatically printed to the screen.

Arguments:   The starting and ending wavenumbers (float).

Returns:     The calculated value (float).  Also the return value is

stored in the z register.

Example:     load "pstyrene.abs"  /* Load file into memory. */

p2pnoise 2000 2250   /* Calculate peak-to-peak noise */

/* between 2000 and 2250 cm-1. */

print z              /* Print the results. */

###

___pause_______________________________________________________________

Description: Use to display a message and wait for the user to press

a mouse key, the Escape key, or the Enter key.  This

command is useful when the user must complete some task

(for example, inserting a sample) before the macro can

continue.

Arguments:   (optional)  Text to display (string).  If no string is

specified, the message "Press any key to continue" is

displayed.

Example:     pause "Insert sample.  Press Enter key when ready."

/* Display message and wait for user response. */

scan  /* Begin sample data collection. */

###

___peakAt______________________________________________________________

Description: Use to get the x and y axis values for the peak closest

to the wavenumber you specify.

Arguments:   The wavenumber for which to get values (float).

Returns:     The x and y axis location (wavenumber and intensity) of

the closest peak (float).  Also, the x axis return

value is stored in the y register and the y axis return

value is stored in the z register.

Example:    load "pstyrene.abs"  /* Load file into memory. */

peakat 1600          /* Find peak nearest 1600. */

###

___peakPercent_________________________________________________________

Description: Use to report a specified y-axis value as a percentage

of the reference peak (established with the peakref

command).  If verbose is set to "on", you can obtain a

peak percentage report for several wavenumbers.

Arguments:   Up to 19 wavenumbers for which you want to perform the

calculation (float).

Returns:     The y axis percentage value for the specified wave-

number or for the last specified wavenumber if more

than one wavenumber is specified (float).  Also, the

return value is stored in the z register.

Example:     load "pstyrene.abs"  /* Load the file into memory. */

verbose "on"         /* Set verbose to "on." */

peakat 3000          /* Get nearest peak location. */

peakref y            /* Set the reference peak to be */

/* the peak at the wavenumber */

/* reported by the peakat */

/* command. */

peakpercent  2000 1943  /* Report the y-axis values */

/* that correspond to 2000 and */

/* 1943 wavenumbers as */

/* percentages of the peak */

/* specified with the peakref */

/* command. */

verbose "off"        /* Set verbose to "off." */

###

___peakRef_____________________________________________________________

Description: Use to specify a reference peak for the peak

percentage calculation.  (See the peakpercent command.)

Arguments:   The wavenumber location of the peak (float).

Returns:     The y-axis value for the peak (float).  The return

value is stored in the z register.  The wavenumber

location of the reference peak is stored in the y

register.

Example:     load "pstyrene.abs"  /* Load the file into memory. */

peakat 3000          /* Retrieve the nearest peak. */

peakref y            /* Set the reference peak to be */

/* the peak at the wavenumber */

/* found by the peakat command. */

peakpercent 1943     /* Report the y-axis value for */

/* the peak at 1943 as a */

/* percentage of the peak */

/* specified with the peakref */

/* command. */

###

___peaks_______________________________________________________________

Description: Use to generate a peak report, and annotate the peaks

for the current sample.

Arguments:   None.

Example:     load "pstyrene.abs"  /* Load file into memory. */

peaks                /* Generate a peak report */

/* and print it to the report */

/* display. */

###

___pickFile____________________________________________________________

Description: Use to display a list of all files in the current

directory that match the DOS filename with optional

wildcards (? and *).  With this list displayed, ApPro

waits for the user to select a listed file.

Arguments:   Any valid DOS filename with optional wildcards, which

include ? and * (string).

Returns:     The name of the file selected or an empty string if

nothing is selected.

Example:     s = pickfile "*.abs"  /* List all files with the */

/* .abs extension, and store */

/* the selected file in the s /

/* register. */

a = stringlength s    /* Place the length of the file */

/* name in s into a. */

if (a > 0)            /* If a file was selected... */

load s               /* Load the file.

###

___pickPoints__________________________________________________________

Description: Use to select points on the displayed data and place

those points into a user-defined array.  The points are

stored in the array as xy pairs (x value, then y

value).  For example, for the first selected point, the

x value is stored in array:data[1], the y value is

stored in array:data[2].  Similarly, for the nth point,

the x value is stored in array:data[ (n * 2) -1 ], and

the y value is stored in array:data[ n * 2 ].)  You

can configure this command several ways, as explained

in the arguments section.  After you define the array,

it is resized, set to 1 dimension, and set to a type of

float.

Arguments:   The user array to add points to (array), followed by

two optional arguments (integers).  (You can use the

first or both optional arguments.  But to use the

second argument, you must use the first argument.)

The first optional argument specifies the number of

points you want to selected for the array.  The second

optional argument sets the type of data cursor to use

as you select points.  If no arguments are specified,

default settings are used: the default number of points

is no limit and the default data cursor is the "sticky"

cursor.  (The cursor sticks to the data trace, to force

the cursor's y coordinate to match the data's y

coordinate at the current wavenumber position.)

If the specified number of points to pick is a positive

integer, the pickpoints command returns after you

select the specified number of points.  (Setting

the first argument to 0 is equivalent to no limit.)

If the second argument is positive (and non-zero), the

data cursor moves freely across the data display.

(Setting the second argument to 0 uses the default

data cursor.)

Returns:     1 if points are picked successfully, 0 if not (int).

Example:     float array datapnts     /* Declare a float array to */

/* hold selected points. */

pickpoints datapnts 0 1  /* Pick any number of data */

/* points with a free- */

/* moving cursor. */

print "You picked these data points:" /* Show text */

/* in the report display. */

/* Execute loop statements starting with x equal to */

/* 1 and increment x by 2 (step 2) at each iteration */

/* of the loop. Stop executing the loop when the */

/* value of x is greater than or equal to the number */

/* of points in the array (datapnts:points). */

for x = 1 to dataPnts:points step 2  /* Loop. */

{           /* Mark the start of the loop. */

print datapnts:data[x]   " , "   datapnts:data[x + 1]

/* Print out the select points using */

/* this format: x value, y value. */

}           /* Mark the end of the loop. */

###

___pks_________________________________________________________________

Description: Pks is an object that contains properties and methods

you can set and execute to generate peak reports.

Properties:  Properties for the pks object are:

pkSort:        Use to specify the peak sorting method.

Arguments:     The name of the peak sorting method you want to

use: frequency or intensity (string).

Syntax:        pks:pkSort = "frequency"

peakMethod     Use to specify the peak report method.

Arguments:     The name of the peak reporting method you want to

use: regression, derivative, lineshape, threshold,

or singles (string).

Syntax:        pks:peakMethod = "lineshape"

peakFilter     Use to specify the peak filtering method to use.

Arguments:     The name of the peak filter you want to use: None,

3point, 5point, or spline (string).

Syntax:        pks:peakFilter = "5point"

peakColumns    Use to specify the number of columns to use in the

peak report.

Arguments:     The word that identifies the number of colums to

use: One, Two, Three or Four (string).

Syntax:        pks:peakColumns = "three"

peakSensitivity Use to specify the number of standard deviations

a data point must be above the baseline to be

considered a peak.

Arguments:     Any value (int).

Syntax:        pks:peakSensitivity = 25

Methods:     Methods for the pks object are:

pick:          Use to generate the peak report.

Arguments:     None.

Syntax:        pks:pick

readMethod:    Use to load a math method (configuration) and put

in place all math parameters it specifies,

including properties for the pks object.

Arguments:     (optional)  The name of the math method to load.

If no name is specified, the Load Math Setup

dialog appears.

Syntax:        pks:readmethod

saveMethod:    Use to save all current math parameters, including

pks object properties, as a math method

(configuration).

Arguments:     (optional)  The name to use for the math method

you save.  If no name is specified, the Save Math

Setup dailog appears.

Syntax:        pks:savemethod

###

___plot________________________________________________________________

Description: Use to plot the current data and/or text using the

specified plot method.

Arguments:   The name of the plot method you wish to use.

Example:     load "pstyrene.abs"  /* Load file into memory. */

plot "plot1.ini"     /* Plot using the plot method */

/* with plot1.ini as its name. */

###

___power_______________________________________________________________

Description: Use to perform exponential multiplication (i.e., raise

x to the power of y).  For example: 2 raised to the

power of 3 is equal to 2 x 2 x 2 = 8.

Arguments:   The base number upon which the exponent acts (float),

the exponent (float).  Even though both values can be

floating point numbers, there are limitations:

- if the base number is negative, the exponent must be

an integer.

- if the base number is 0, the exponent must be greater

than 0.

Returns:     The calculated value (float).

Example:     print (power 2.5 3.5)  /* Raise 2.5 to the power of */

/* 3.5 and print result */

/* on report display. */

###

___precision___________________________________________________________

Description: Use to set the number of decimal places to report.

Arguments:   The number of decimal places to report (int).

Example:    load "pstyrene.abs"  /* Load file into memory. */

precision 3          /* Set the number of decimal */

/* places to show when printing */

/* a report. */

peaks                /* Generate a peak report. */

###

___predictX____________________________________________________________

Description: Use to predict the x value given a specific y value.

(Use this command after establishing a data points

file and using the calibrate command to calibrate that

file.)  Also, if verbose is set to "on," the predictX

command creates a report of the predicted x value for

each of for several given y values.

Arguments:   Up to 19 y values (float).

Returns:     The predicted value (float).  Also the return value is

stored in the z register.  If more than one y value is

given, the predicted x value for the last y value is

returned.

Example:     calibrate  "pairs.dat"  /* Do a linear regression */

/* on the data points in */

/* the pairs.dat file. */

predictx 2.5            /* Based on the calibration */

/* results, predict x when */

/* 2.5 is the y value. */

###

___predictY____________________________________________________________

Description: Use to predict the y value given a specific x value.

(Use this command after establishing a data points

file and using the calibrate command to calibrate that

file.)  Also, if verbose is set to "on," the predictY

command creates a report of the predicted y value

for each of several given x values.

Arguments:   Up to 19 x values (float).

Returns:     The predicted value (float).  Also the return value is

stored in the z register.  If more than one x value is

given, the predicted y value for the last x value is

returned.

Example:     calibrate  "pairs.dat"  /* Do a linear regression */

/* on the data points in */

/* the pairs.dat file. */

predicty 2.5            /* Based on the calibration */

/* results, predict y when */

/* 2.5 is the x value. */

###

___print_______________________________________________________________

Description: Use to display text in the Genesis report display.  The

arguments are concatenated into a text string and then

printed to the screen.  When a variable or register is

used as an argument, the value stored within that

variable or argument is printed.

Arguments:   Any literal value, register, or variable.  (Up to 19

arguments are allowed.)

Example:     calibrate  "pairs.dat"  /* Do a linear regression */

/* on the data points in */

/* the pairs.dat file. */

predictx 2.5            /* Based on the calibration */

/* results, predict x when */

/* 2.5 is the y value. */

print "When y = 2.5, the predicted x value is:  "  z

/* Print the results.  In this case, there are two */

/* arguments: a text string which is printed exactly */

/* as it appears when typed, followed by a register */

/* that, in this example, contains the results from */

/* the predictx command.  The register contents, not */

/* the letter z, are printed after the text string. */

###

___quit________________________________________________________________

Description: Use to exit the Genesis software.

Arguments:   None.

Example:     quit  /* Exit the Genesis software. */

###

___ras2abs_____________________________________________________________

Description: Use to convert transmittance data (the current sample)

to absorbance data.

Arguments:   None.

Example:     load "trans.ras"  /* Load file into memory. */

ras2abs           /* Convert data to absorbance. */

show              /* Display the absorbance data. */

###

___ras2drt_____________________________________________________________

Description: Use to convert transmittance data (the current sample)

to diffuse reflectance data.

Arguments:   None.

Example:     load "trans.ras"  /* Load file into memory. */

ras2drt           /* Convert data to diffuse */

/* reflectance. */

show              /* Display the results. */

###

___ratio_______________________________________________________________

Description: Use to ratio sample single beam data with another

single beam data file (typically a background file).

Arguments:   (optional)  The name of the background single beam

(string).  If a filename is not included, the current

background data is used.

Example:     load "pstyrene.sbm"  /* Load file into memory. */

ratio "sample.bkg"   /* Ratio the pstyrene.sbm file */

/* with the sample.bkg file. */

show                 /* Display the results. */

###

___reload______________________________________________________________

Description: Use to reload the current sample from disk.  All

unsaved data changes are discarded.

Arguments:   None.

Example:     load "pstyrene.abs"  /* Load file into memory. */

abs2ras              /* Convert the data to */

/* transmittance. */

save "pstyren1.ras"  /* Save the transmittance data. */

reload               /* Load pstyrene.abs again. */

###

___rename______________________________________________________________

Description: Use to change the name of a file.  (After using this

command, the file no longer exists under its old name.)

Arguments:   The original name of the file (string), followed by the

name of the new file (string).

Returns:     1 if the file has been renamed, 0 if not (int).

Example:     rename "pstyrene.abs"  "pstyrene.bak"

/* Rename the file originally named pstyrene.abs */

/* as a file with pstyrene.bak as its name. */

###

___rewind______________________________________________________________

Description: Use to move to the beginning of a text or binary file

that is opened for read and write.

Arguments:   None.

Returns:     1 if successful, 0 if not (int).

Example:     fopen 1 "binary.dat"  /* Open binary.dat as file 1. */

fwrite 1 2.5          /* Write 2.5 to open file. */

fwrite 1 2300         /* Write 2300 to open file. */

rewind 1              /* Move to top of file 1. */

fread 1 a             /* Read first value into the a */

/*  register. */

fread 1 b             /* Read next value into the b */

/* register. */

print  a "," b        /* Print the read values. */

fclose 1              /* Close file 1. */

###

___rmsNoise____________________________________________________________

Description: Use to calculate the root mean squared noise of the

current spectrum within a specified wavenumber range.

If verbose is set to "on," this value is automatically

printed to the screen.

Arguments:   The starting and ending wavenumbers (float).

Returns:     The calculated value (float).  Also the return value is

stored in the z register.

Example:     load "pstyrene.abs"  /* Load file into memory. */

rmsnoise  2000 2250  /* Find root mean squared noise */

/* between 2000 and 2250 cm-1. */

print z              /* Display the results. */

###

___run_________________________________________________________________

Description: Use to execute an ApPro macro.  You can use this

command from the command line or from within a macro.

NOTE: The difference between the run command and the

include command is that the run command lets you "pass"

command line arguments to the macro you execute.  That

is, you can place specific values on the same line with

the run command.  It is the programmer's responsibility

to retrieve these arguments from the command line

argument structure before using run to call another

file which also uses command line arguments.  Command

line arguments with the run command are optional.

Arguments:   The name of the macro file to execute (string),

followed by as many as 8 command line arguments.  The

command line arguments can be any literal value, any

register, and any variable.

Example 1:   load "pstyrene.abs"  /* Load file into memory. */

run "myNeta.mac" 3000 2500 2000  /* Execute the */

/* macro, and pass it 3 command line arguments. */

Example 2:   /* "Analysis.mac" */

x = cmdargs:args[1]  /* Place the first command */

/* line argument in register x. */

y = cmdargs:args[2]  /* Place the second command */

/* line argument in register y. */

z = cmdargs:args[3]  /* Place the third command */

/* line argument in register z. */

neta x y z           /* Calculate net absorbance for */

/* the values stored in registers x, y, and z. */

###

___save________________________________________________________________

Description: Use to save the current sample data to disk.

Arguments:  (optional) Name to assign to the saved file (string).

If you do not specify a name, the file is saved using

the original file name (and original file is replaced).

Returns:    1 if the operation is successful, 0 if not (int).

Example:    load "pstyrene.abs"  /* Load file into memory. */

abs2ras              /* Convert the data to */

/* transmittance. */

save "pstyren1.ras"  /* Save the transmittance data. */

###

___scale_______________________________________________________________

Description: Use to scale the sample data by a specified value.

Each data point is multiplied by the specified scaling

factor.

Arguments:   The number to use as the scaling factor (float).

Example:     load "pstyrene.abs"  /* Load the file into memory. */

scale 2.5            /* Scale data using 2.5 as */

/* the scaling factor. */

show                 /* Show the results. */

###

___scaleTo_____________________________________________________________

Description: Use to scale the sample data to a specific value.  The

maximum y value in the sample data is scaled to equal

the value you specify, and all other data values are

then scaled by that same amount.

Arguments:  The value to you want to scale to (float).

Example:    load "pstyrene.abs"  /* Load file into memory. */

scaleto 3.4          /* Scale the maximum y value to */

/* make is equal 3.4 and scale */

/* all other values by that */

/* same amount. */

show                /* Display the results. */

###

___scaleToPeak_________________________________________________________

Description: Use to scale all reference data files in the active

data window to a sample data peak you specify.

Arguments:   The wavenumber location of the sample data peak to

which you want to scale reference data files (int).

Example:     load "tester.abs"  /* Load file into memory. */

scaletoPeak 3205.4  /* Scale all reference data */

/* files to the sample data peak */

/* at 3205 wavenumbers.*/

show                /* Display the results. */

###

___scan________________________________________________________________

Description: Use to collect sample data from the spectrometer.

Arguments:   None.

Example:     ask "Do you want to collect a sample?"

/* Ask question and wait for response. */

if (z == 1)  /* If answer is y, read next line. */

scan        /* Collect sample data. */

###

___search______________________________________________________________

Description: Use to perform a library search to find the best

matches for the sample spectrum.

Arguments:   (optional)  The name of the library you want to search

(string).  If you do not specify a name, the default

library (icon-1) is searched.

Example:     load "tmp.abs"   /* Load file into memory. */

search "icon-1"  /* Compare the sample data with */

/* spectra in the icon-1 library */

/* to find the best matches. */

###

___setStringElement____________________________________________________

Description: Use to set an individual character within a string.

Arguments:   The string in which you want to set the character

(string), position of the character you want to set

(int), the value to replace the character at the

specified position (int or char).  If the value to put

in the string is expressed as an integer, that integer

also serves as the decimal representation for the ASCII

character.

Returns:   The new text string (string).

Example:   x = "This is a string"  /* Place text in x register. */

y = setStringElement x  14 'i'  /* Replace the 14th */

/* character in the string */

/* with the letter i. */

###

___shell_______________________________________________________________

Description: Use to create a DOS shell (command.com from Windows) in

which you can enter DOS commands.  This is a separate

instance of DOS, and all DOS commands are valid.  To

leave the DOS shell, type exit.

Arguments:   None.

Example:    shell  /* Start a DOS shell through Windows.  Enter */

/* DOS commands as needed.  Type exit to */

/* return to ApPro. */

###

___show________________________________________________________________

Description: Use to display the data display area (and any loaded

data).

Arguments:   None.

Example:     load "pstyrene.abs"  /* Load file into memory. */

peaks                /* Generate and display a */

/* peak report. */

show                 /* Display the data again. */

###

___showBitmap__________________________________________________________

Description: Use to display a bitmap in the Genesis software.  The

bitmap is displayed in dimensions proportional to the

original.  To instead fit the bitmap to the Genesis

display area, use the stretchbitmap command.

Arguments:   The name of the bitmap file to display (string).

Returns:     1 if file loads correctly, 0 if not (int).  Also, the

return value is stored in the z register.

Example:     showbitmap "pic.bmp"  /* Display the pic.bmp bitmap. */

###

___showFile____________________________________________________________

Description: Use to display a text file in the report area.

Arguments:   The name of the file to display (string).

Returns:     1 if file loads correctly, 0 if not (int).  Also, the

return value is returned in the z register.

Example:     showfile "file1.dat"  /* Display the file.data text */

###

___sin_________________________________________________________________

Description: Use to compute the sine of the specified value.

Arguments:   Value (in radians) for which you want the sine (int or

float).

Example:     y = sin 0  /* Place the sine of 0 in y register. */

x = sin z  /* Place sine of value stored in z */

/* register into x register. */

###

___sleep_______________________________________________________________

Description: Use to stop a program (i.e., a macro) for as many

seconds as you specify.  All Genesis and ApPro input

is also "asleep."

Arguments:   The number of seconds to sleep (int).

Example:     sleep 5  /* Sleep for 5 seconds. */

###

___smooth______________________________________________________________

Description: Use to smooth data using the Savitsky-Golay method.

Arguments:   Any odd number of smoothing points (int).

Example:     load "co.abs"  /* Load file into memory. */

smooth 9       /* Perform Savitsky-Golay smooth. */

show           /* Display the results. */

###

___smth________________________________________________________________

Description: Smth is an object that contains properties and methods

you can set and execute to perform spectral data

smoothing.

Properties:  Properties for the smth object are:

smoothPoints:  Use to specify the number of smooth point to use.

Arguments:     An odd number within the legal range for the

specified smooth method: for boxcar, 3 to 25;

for cubic or quadratic, 5 to 25; and for quartic

or quintic, 7 to 25.  Specified even numbers are

forced to the next lower odd number.  Any number

outside the legal range is forced within range

(int).

Syntax:        smth:smoothPoints = 15

smoothMethod:  Use to specify the smoothing method.

Arguments:     The name of the smoothing method to use: boxcar,

cubic, quadratic, quartic, or quintic (string).

Syntax:        smth:smooth method = "cubic"

Methods:          Methods for the smth object are:

process:       Use to perform the smooth.

Arguments:     None.

Syntax:        smth:process

readMethod:    Use to load a math method (configuration), and

put in place the math parameters it specifies,

including smth object properties.

Arguments:     (optional)  The name of the math method to load.

If you specify no method, the Load Math Setup

dialog appears.

Syntax:        smth:readMethod

saveMethod:    Use to save a math method (configuration),

including all current smth object properties.

Arguments:     (optional)  The name of the math method to save.

If you specify no method, the Save Math Setup

dialog appears.

Syntax:        smth:saveMethod

###

___spc2matt____________________________________________________________

Description: Use to a translate a copy of a file in the Spectral

Calc (Grams) format to a file in the ATI Mattson FIRST

data file format.  The translated file becomes the

current sample.  The original Spectral Calc file that

is translated is not removed or otherwise altered by

the spc2matt command.  You can use the translated data

file in the same way you would any data file created by

WinFIRST.

Arguments:   The directory path and name of Spectra Calc file

(string).

Example:     spc2matt "\scalc\data\pstyrene"

/* Translate file and make it the */

/* sample data file. */

###

___sqrt________________________________________________________________

Description: Use to compute the square root of the specified value,

or the square root of the current sample if no value is

specified.

Arguments:   (optional)  The value for which to calculate square

root (int or float).  This value must be positive.

Returns:     If a value is given, the square root of the specified

value (float).  If a non-positive argument is given,

0 is returned (int).

Example 1:   load "pstyrene.abs"  /* Load file into memory. */

sqrt                 /* Compute the square root */

/* of the sample data. */

show                 /* Display the results. */

Example 2:   sqrt 81  /* Compute the square root of 81. */

###

___square______________________________________________________________

Description: Use to compute the square of the specified value, or

the square of the current sample if no value is

specified.  When the data is squared, only positive

values are squared.

Arguments:   (optional)  The value to square (float).

Returns:     If a value is given, the square of the specified

value (float).

Example:     load "pstyrene.abs"  /* Load file into memory. */

square               /* Calculate the square of */

/* the sample data. */

show                 /* Display the results. */

Example 2:   square 2.5           /* Calculate square of 2.5. */

###

___stack_______________________________________________________________

Description: Use to display the current sample and reference data in

the stack display style.  (The current sample is

displayed at the bottom of the data display area.  The

R1, or first, reference appears just above sample data

with the R2 reference above R1, R3 above R2 and R4

above R3.)  If a file name is provided as an argument,

that data file is added to the reference stack as the

R1 reference.  If you add a new reference with this

command, the data is displayed.  Otherwise, the next

time you display data it is displayed using the stack

display style.

Arguments:   (optional)  The name of the file to add to the

reference stack (string).

Example:     load "mix2.abs"  /* Load file into memory. */

stack "mix.abs"  /* Stack mix2.abs with mix.abs */

/* and display the results. */

###

___statusMsg___________________________________________________________

Description: Use to display a message at the ApPro command line.

After the message is displayed, the macro immediately

continues running.

Arguments:   Up to 19 valid ApPro values: register, variable, or

literal values.

Example:     fopen 1 sample.txt  /* Open sample.txt as file 1. */

fread 1 a           /* Read first value to the a */

/* register. */

eof 1               /* Ask if at end of the file. */

if (z == 1)         /* If at end of file... */

statusMsg "End of file reached.. "  /* Show message. */

else                /* If not at end of file... */

fread 1 b          /* Read second value. */

fclose 1            /* Close the file. */

###

___stretchBitmap_______________________________________________________

Description: Use to display a bitmap in the Genesis software.  The

bitmap is "stretched" to fill the Genesis display area.

To instead maintain the original bitmap proportions,

use the showbitmap command.

Arguments:  The name of the bitmap file to display (string).

Returns:    1 if file loads correctly, 0 if not (int).  Also, the

return value is stored in the z register.

Example:    stretchbitmap "pic.bmp"  /* Display the bitmap named */

/* pic.bmp and adjust its */

/* dimensions to fit the */

/* Genesis display area. */

###

___string2Float________________________________________________________

Description: Use to convert a text string to a floating point

number.

Arguments:   The text to convert (string).

Returns:     The floating point number that is expressed by the

specified text (string).

Example:     /* This reads text from a file, and converts the */

/* text into a floating point number: */

fopen 1 "testfile.dat"  /* Open testfile.dat file. */

finput 1 a              /* Read first line in file */

/* and store in register a. */

fclose 1                /* Close the file. */

f = string2float a      /* Convert text in the a */

/* register to a floating */

/* point number and store */

/* in the f register. */

print "The first value in testfile.dat is " f

/* Print the results in the */

/* Genesis report area. */

###

___string2Int__________________________________________________________

Description: Use to convert a text string to an integer.

Arguments:   The text to convert (string).

Returns:     The integer that is expressed by the text (string).

Example:     /* This reads text from a file, and converts the */

/* text into an integer: */

fopen 1 "testfile.dat"  /* Open testfile.dat file. */

finput 1 a              /* Read first line in file */

/* and store in register a. */

fclose 1                /* Close the file. */

i = string2int a        /* Convert text in the a */

/* register to an integer */

/* and store the value in */

/* the i register. */

print "The first value in testfile.dat is " i

/* Print the results in the */

/* Genesis report area. */

###

___stringCompare_______________________________________________________

Description: Use to tell if two text strings have the same content

(are equal to one another).

Arguments:   The two text values to compare (string).

Returns:     1 if the text strings are identical, 0 if not (int).

Example:     enter n "Type name of file to load (or q to abort)"

/* Get the file name and place it */

/* in the n register. */

a = stringcompare n "q"  /* Compare text in register */

/* n with the string "q" and */

/* store the return value in */

/* the a register. */

if (a == 0)      /* If the strings are not equal ... */

load n           /* Load the file named in the n */

/* register. */

###

___stringIndex_________________________________________________________

Description: Use to find the string index (character position) for

a specified string within a string.

NOTE: This command is letter-case sensitive.

Arguments:   The string to search for (string), followed by the

string in which to search.

followed by the string .

Returns:     The first index position of the located string or 0 if

the string was not located (int).

Example:     local int index  /* Declare local integer variable. */

index = stringIndex "Pro" "ApPro"

/* The value 3 is assigned to the index variable. */

###

___stringLeft__________________________________________________________

Description: Use to extract part of the left end of a text string.

The stringleft command builds a new text string by

extracting a specified number of characters, starting

from the leftmost character.

Arguments:   The string from which to extract text (string),

followed by the number of characters to extract (int).

Returns:     The extracted text (string).

Example:     /* This extracts the day from the result of the time */

/* command and sets the title of the loaded sample */

/* to the current day:*/

string day  /* Declare a string variable to hold the */

/* text that represents the current day. */

t = time    /* Get the date and time, and place them */

/* in the t register. */

day = stringleft t 10  /* Get the day (10 leftmost */

/* characters) from the string */

/* stored in the t register. /*

load "tmp.abs"  /* Load file into memory. */

title day       /* Set title for tmp.abs to match */

/* string stored in day variable. */

###

___stringLength________________________________________________________

Description: Use to get the length (in number of characters) of a

specified string.

Arguments:   The string whose length you want to get (string).

Returns:     The length of the string (int).

Example:     /* This asks the user to enter a file name to use */

/* to save newly acquired sample data: */

pause "Insert sample.  Press Enter key when ready."

/* Ask user to insert the sample */

/* and then continue when ready. */

scan           /* Collect a sample. */

enter n "Enter file name to use for this sample."

/* Ask user for new file name and */

/* store in register n. */

l = stringlength n  /* Get name length in register */

/* n and store in register l. */

/* (Eight characters is maximum */

/* name length DOS allows.) */

if ( l > 8 )        /* If length is more than 8... */

a = stringleft n 8  /* Set name to use equal first */

/* 8 characters only. */

save a              /* Save the new data using name */

/* stored in register a. */

###

___stringLower_________________________________________________________

Description: Use to make all characters in a string lowercase.

Arguments:   The string upon which to act (string).

Example:     s = stringLower "ApPro"

print s   /* The following string is printed: appro. */

###

___stringMid___________________________________________________________

Description: Use to extract part of the middle of a text string.

The stringmid command builds a new text string by

extracting a specified number of characters, starting

at a specified character position.  The starting

position is specified by its distance (in characters)

from the left of the text string.

Arguments:   The string from which to extract the text (string),

followed by the starting position (int), followed by

the number of characters to extract (int).

Returns:     The extracted text (string).

Example:     /* This extracts the local time from the result of */

/* the time command and sets the title of the loaded */

/* sample to the current local time: */

string localtime  /* Declare a string variable to */

/* hold text that represents the */

/* current time. */

t = time          /* Get the date and time, and */

/* store in the t register. */

localtime = stringmid t 12 8  /* Get the 8 middle */

/* characters (time) */

/* from the string in */

/* the t register and */

/* store in localtime. */

load "tmp.abs"    /* Load file into memory. */

title localtime   /* Set title for tmp.abs to the */

/* string in localtime variable. */

###

___stringRight_________________________________________________________

Description: Use to extract part of the right side of a text string.

The stringright command builds a new text string by

extracting the right part of a string starting at the

specified character position, and continuing to the end

of the string.  The specified starting position is

specified by its distance (in characters) from the

right of the text string.

Arguments:   The string from which to extract text (string),

followed by the starting character position (int).

Returns:     The extracted text (string).

Example:     /* This extracts the year from the result of the */

/* time command and sets the title of the loaded */

/* sample to the current year: */

string year  /* Declare a string variable to hold */

/* text that represents current year. */

t = time     /* Get the date and time, and store */

/* in the t register. */

year = stringright t 4  /* Get the 4 rightmost */

/* characters in the string held in the */

/* t register (year) and place in year. */

load "tmp.abs"  /* Load file into memory. */

title year   /* Set title for tmp.abs to string */

/* stored in year variable. */

###

___stringUpper_________________________________________________________

Description: Use to make all characters in a string uppercase.

Arguments:   The string upon which to act (string).

Example:     s = stringUpper "ApPro"

print s   /* The following string is printed: APPRO. */

###

___subtract____________________________________________________________

Description: Use to subtract reference data (of the same resolution)

from the current sample data.

Arguments:   (optional) Name of the file to use as a reference

(string), followed by a subtraction factor (float).

You must supply a subtraction factor, but if you give

no file name, the subtract command uses the R1

reference.

Returns:     1 if a supplied reference name loads correctly, 0 if

not (int).

Example:     load "pstyrene.abs"  /* Load file into memory. */

subtract "h2O.abs" 0.5  /* Subtract 0.5 of h20.abs */

/* from sample data. */

show                   /* Display results. */

###

___superimpose_________________________________________________________

Description: Use to superimpose loaded references over sample data.

If a file name is specified, it is loaded as the R1

reference.

Arguments:   (optional)  The name of the new reference (string).

Example:     load "mix.abs"          /* Load file into memory. */

superimpose "mix2.abs"  /* Load mix2.abs and */

/* superimpose on sample. */

###

___system______________________________________________________________

Description: Use to run another DOS or Windows executable program

(application). To return to Genesis, you must

terminate (quit) the application.

Arguments:   The command line that starts the application (string).

Usually this consists of a program name and 0 or more

command line arguments.

Example:     system "pbrush.exe"  /* Run Windows Paintbrush, and */

/* wait for user to quit it. */

###

___tan_________________________________________________________________

Description: Use to compute the tangent of the specified value.

Arguments:   The value (in radians) for which to calculate the

tangent (int or float).

Example:     y = tan 0  /* Place the tangent of 0 in register y. */

x = tan z  /* Place tangent of value in register z */

/* into register x. */

###

___tilewindows______________________________________________________

Description: Use to arrange all open data and report windows as

tiles.

Arguments:   None.

Example:     load pure.abs     /* Load data file. */

peaks             /* Generate peak report. */

load pure2.abs    /* Load data file. */

peaks             /* Generate peak report. */

load pstyrene.abs /* Load data file. */

peaks             /* Generate peak report. */

tilewindows       /* Arrange all open data and */

/* report windows as tiles. */

###

___time________________________________________________________________

Description: Use to get the system (computer) date and time.

Arguments:   None.

Return:      The system date and time (string).  For example:

"Fri Jan 01 15:45:01 1993"

Example:     t = time    /* Get the system time and store in */

/* register t. */

print t     /* Print the system time. */

###

___timercheck__________________________________________________________

Description: Use to find the number of seconds that have elapsed

since the timer was started, with the timerstart

command.

Arguments:   None.

Return:      The number of seconds (int).  Also, the return value

is stored in the z register.

Example:     int secs  /* Declare an integer variable to hold */

/* the number of seconds. */

ask "Do you want to collect a sample?"

if (z == 1)  /* If answer is y... */

scan        /* Collect sample data. */

else         /* Otherwise... */

exit        /* Exit the macro. */

timerstart   /* Start the timer. */

enter n "Enter a name for this sample."

/* Ask for sample name and put in n. */

secs = timercheck  /* Get elapsed time and store */

/* in the secs variable. */

if (secs > 3600)   /* If time is more than 1 hour... */

{

enter a "This sample is too old.  Save it?" */ Ask. */

if (z == 1)        /* If answer is y... */

save  n           /* Save with name in n. */

}

else         /* Otherwise (time less than 1 hour)... */

save n      /* Save automatically with name in n. */

###

___timerStart__________________________________________________________

Description: Use to start the timer.  (Use the timercheck command to

monitor the number of seconds elapsed since the timer

was started.)  Also, use the timerstart command to

reset (restart) the timer.

Arguments:   None.

Example:     int secs  /* Declare an integer variable to hold the */

/* number of elapsed seconds. */

ask "Do you want to collect a sample?"  /* Ask. */

if (z == 1)  /* If answer is y... */

scan        /* Collect sample data. */

else         /* Otherwise (answer is not y)... */

exit        /* Exit the macro. */

timerstart   /* Start the timer. */

enter n "Enter a name for this sample."

/* Ask for a file name for the sample. */

secs = timercheck  /* Get the elapsed time and store */

/* in the secs variable. */

if (secs > 3600)   /* If more than 1 hour elapsed... */

{

enter a "This sample is too old.  Save it?"  */ Ask. */

if (z == 1)        /* If answer is y... */

save  n           /* Save sample with name in n. */

}

else         /* Otherwise (time less than 1 hour)... */

save n      /* Save automatically with name in n. */

###

___timeStampString_____________________________________________________

Description: Use to get the time stamp for the active sample data

file.

Returns:     The sample data file's time stamp as a string (string).

Arguments:   None.

Example:     t = buildString timeStampString

/* Place the string returned by */

/* timeStampString into the t register. */

s = buildString "Unknown compound collected on" t

/* Place the new string returned by */

/* buildString into the s register. */

title s    /* Set the data file title to be the */

/* string in register s. */

###

___timeStampValue______________________________________________________

Description: Use to get the time stamp for the active sample data

and convert that value to a string.

Returns:     The sample data file's time stamp (int).

Arguments:   None.

Example:     int function showFileTime (string file1, string file2)

{

local int t1

local int t2

local float tdiff

local string f1

local string f2

if(load file1)

{

t1 = timeStampValue

if(load file2)

{

t2 = timeStampValue

if(t2>=t1)

{

f1 = buildString file1

f2 = buildString file2

tdiff = (t2 - t1)

}

else

{

f1 = buildString file2

f2 = buildString file1

tdiff = (t1 - t2)

}

tdiff = (tdiff / 100.0)

m = 0.0

h = 0.0

d = 0.0

m = (60)

h = (60 * m)

d = (24 * h)

D = 0

while(tdiff>=d)

{

tdiff = (tdiff - d)

increment D

}

H = 0

while(tdiff>=h)

{

tdiff = (tdiff - h)

increment H

}

M = 0

while(tdiff>=m)

{

tdiff = (tdiff - m)

increment M

}

S = (int) tdiff

s = buildString f1" was collected "D" days "H" hours "M"

minutes and "S" seconds after "f2

print s

}

}

}

###

___title_______________________________________________________________

Description: Use to assign a title for the current sample.  (To save

the title with the sample data, use the save command.)

Arguments:   The title to be associated with the spectrum (string).

Example:     load "tmp.abs"     /* Load file into memory. */

title "sample1  Fri, Jan 1, 1993"  /* Assign a title */

/* to the sample. */

save "samp1a.abs"  /* Save sample (with title) as */

/* samp1a.abs. */

###

___truncate____________________________________________________________

Description: Use to extract a wavenumber (or data point) region from

the sample spectrum (or sample interferogram).

Arguments:   The starting and ending wavenumbers (or data points)

for the range you want to extract (float).

Example:     load "pstyrene.abs"  /* Load file into memory. */

truncate 3000 2900   /* Cut off data outside 3000 */

/* and 2900 wavenumbers. */

show                 /* Display the resulting data. */

###

___value_______________________________________________________________

Description: Use to get the y value for a specified wavenumber.

Also, if verbose is set to "on", the value command

supplies report of the y values for several given

wavenumbers.

Arguments:   Up to 19 wavenumbers for which you want y values

(float).

Returns:     The y value for the given wavenumber (float).  Also,

the return value is stored in z register.  (If more

than 1 wavenumber value is specified, the y value for

of the last wavenumber is returned.)

Example:     load "pstyrene.abs"   /* Load file into memory. */

verbose "on"          /* Set verbose to "on." */

value 2000 3000 3500  /* Get report of y values. */

/*  Also, place y value for */

/* 3500 in the z register. */

verbose "off"         /* Set verbose to "off." */

###

___verbose_____________________________________________________________

Description:  Use to control the amount of information displayed by

certain commands.

Arguments:    An "on" setting allows added information, any other

setting specifies minimal information (string).

Example:      load "pstyrene.abs"  /* Load the file into memory */

verbose "on"         /* Set verbose to "on." */

peakat 3000          /* Retrieve the nearest peak. */

peakref y            /* Set the reference peak to */

/* be the wavenumber retrieved */

/* by the peakat command. */

peakpercent  2000 1943  /* Report the y-axis values */

/* that correspond to 2000 and */

/* 1943 wavenumbers as */

/* percentages of the peak */

/* specified with the peakref */

/* command. */

verbose "off"       /* Set verbose to "off." */

###

___version_____________________________________________________________

Description: Use to get information for your current version of

Genesis software.

Arguments:   None.

Returns:     Version information (string).

Example:     z = version  /* Get version information and place */

/* it in the z register. */

print z      /* Print the information in z to the */

/* screen. */

###

___xy2matt_____________________________________________________________

Description: Use to read a table of xy values, convert it to a

specified data type, and display it as the current

sample.  If a data type is not specified, ApPro prompts

for one.

Arguments:   The name of the file that contains the xy values

(string), followed by a number that represents a data

type (int), where 1 = abs, 2 = ras, 3 = drt, 4 = sbm,

5 = bkg, 6 = igm, and 7 = big.

Returns:     0 if unsuccessful (int).

Example:     xy2matt pstyrene.tbl 1  /* Load table (pstyrene.tbl) */

/* as an absorbance file. */

###

___annotate____________________________________________________________

Description: Annotate is an object that contains properties and

methods you can set and execute to perform various

annotation operations.

Properties:  Properties for the annotate object are:

on:            Use to make existing annotations visible on the

data display.

Arguments:     None.

Syntax:        annotate:on

off:           Use to make existing annotations invisible on the

the data display.

Arguments:     None.

Syntax:        annotate:off

Methods:          Methods for the annotate object are:

list:          Use to list the annotations for the current sample.

Arguments:     None.

Syntax:        annotate:list

clear:         Use to remove a specified annotation.  (The

annotation is specified by a number that

represents the annotations location in the list

displayed using the list method.)

Arguments:     The number of the annotation to remove (int).  If

no value is specified, or if 999 is specified, all

annotations are removed.

Syntax:        annotate:clear 1   /* Clear first annotation. */

###

___bench_______________________________________________________________

Description: Bench is an object that contains properties and

methods that you can set and execute to perform

various spectrometer functions.

Properties:  You can get the current setting for any property

specified by bench.  For example:

r = bench:method_name  /* Store current setting for */

/* the method_name property */

/* in the r register. */

You can also set a new setting for any property

specified by bench.  For example:

bench:method_name = "scnmeth1"  /* Set setting for */

/* method_name property to */

/* scnmeth1. */

Properties for the bench object are:

background_scans: The number of background scans to collect.

Arguments:     The number of scans (int).

baud_rate:     The computer baud rate to use for computer-

spectrometer communication.

Arguments:     The baud rate (int).

benchfft:      Specify where the FFT is performed.

Arguments:     1 for FFT on spectrometer, 0 for FFT on computer

(int).

communications_port: The communications port to use for data

collection.

Arguments:     The number that represents the port (int), where

1 = COM1, 2 = COM2, 3 = COM3 and 4 = COM4.

data_directory_mode  The data directory to use to store data.

Arguments:     The number that represents the directory to use,

where:

0 = the current month directory (jan, feb, etc.).

1 = a directory with the same name as the scan

method.  If the directory does not exits, it

is created.

2 = directory specified by the method (originally

specified for the method with the cwd property

of the parms object).  If the directory does

not exist, it is created.

data_type:     The type of data you want to collect.

Arguments:     The number that represents the data type (int),

where 0 = abs, 1 = ras, 2 = drt, 3 = sbm, 4 = bkg,

5 = igm and 6 = big.

ending_frequency: The data's ending wavenumber.

Arguments:     A wavenumber (float).

file_naming:   The file naming mode to use.

Arguments:     A number that represents the naming mode (int),

where:

0 = None.  (Scan file name is used without

increment value.)

1 = Prompt for entry of name (during collection).

2 = Auto increment.  (The name specified with the

scan_file_name property and an increment are

used to assemble filename.)

4 = Default.  (An eight-character filename is

assembled using month, day and time of day.

For example, 01081630.abs was created on Jan.

8 at 4:30 p.m.)

fvelindex:     The forward scan velocity.  NOTE: For Genesis FTIR

spectrometers, use only index 2 (6.25 KHz) or 4

(25 KHz).  WARNING: When using WinFIRST software

with ATI Mattson spectrometers other than Genesis,

refer to settings available in the software for

foward scan velocity.  (Use the Advanced

Spectrometer Setup dialog from the WinFIRST

control panel.)

Arguments:     The velocity index to use.

increment:     The number to use as the starting increment when

Auto increment file naming is in use.

Arguments:     The starting increment value (int).

lpfindex:      Use to select the internal or external lowpass

filter.  NOTE: For Genesis FTIR

spectrometers, for a scan velocity of 6.25 KHz,

the internal filter.  For a scan velocity of 25

KHz (and with an MCT detector in use), use an

external filter.

Arguments:     The filter index to use, where: 0 = internal and

1 = external (int).

method_name:   The scan method name.

Arguments:     The method filename (string).

prompt_for_title: Specify whether or not you are prompted to

enter a sample data file title.

Arguments:     1 to enable or 0 to disable (int).

realtime-coadd_mode: Specify whether co-add stops at a specified

number of scans or at a specified signal-to-noise

threshold.

Arguments:     0 for number of scans, 1 for snr threshold (int).

reference_time_threshold: The length of time, in minutes, beyond

which the background spectrum is considered

invalid.

Arguments:     A number to indicate threshold in minutes (int).

resolution:    The resolution of the spectral data (in

wavenumbers).

Arguments:    The wavenumber resolution (float).  Valid values

are: 1.0, 2.0, 4.0, 8.0, 16.0, 32.0, 64.0, and

128.0 wavenumbers.

rvelindex:     The reverse scan velocity.  NOTE: For Genesis FTIR

spectrometers, use only index 2 (6.25 KHz) or 4

(25 KHz).  WARNING: When using WinFIRST software

with ATI Mattson spectrometers other than Genesis,

refer to settings available in the software for

reverse scan velocity.  (Use the Advanced

Spectrometer Setup dialog from the WinFIRST

control panel.)

Arguments:     The velocity index to use.

sample_scans:  The number of sample scans to collect.

Arguments:     The number of scans (int).

saverawdata:   Specify whether raw interferogram data is saved.

Arguments:     1 to save raw data, 0 to not save raw data (int).

scan_file_name: The base name to use with Auto increment file

naming.

Arguments:     The name to use (string).

scan_mode:     The data collection mode to use.

Arguments:     The number that represents the scan mode (int),

where 0 = Standard FTIR, 1 = COADD FTIR, 2 =

Standard Microscope, 3 = COADD Microscope.

NOTE: 2 and 3 are available only in Advanced

Genesis.

scanpromptlevel: The amount of promptinh that takes place

during data collection.

Arguments:     The number that represents the scan promt

level, where: 0 = low and 1 = high (int).

scanverbositylevel: The amount of informational messages that

are reported.

Arguments:     The number that represents the scan verbosity

level, where: 0 = silent (no messages), 1 = brief

(some messages), and 2 = normal (all messages)

(int).

signal_threshold: The signal-to-noise threshold to use (if the

realtime_coadd_mode property is set with 1 as its

argument).

Arguments:     A number to serve as the threshold (int).

starting_frequency: The data's starting wavenumber.

Arguments:     A wavenumber (float).

tune_points:   The total number of data points to collect as a

tune interferogram.

Arguments:     The number of data points (int).

useshuttle:    Specify whether a sample shuttle is in use.

Arguments:     1 to use shuttle, 0 to not use shuttle (int).

writesample:   Specify whether the collected sample is saved.

Arguments:     1 to save sample, 0 to not save sample (int).

writebkg:      Specify whether the collected background is saved.

Arguments:     1 to save background, 0 to not save background

(int).

Methods:    Methods for the bench object are:

bkg:           Use to collect a background (using the current--

most recently loaded--data collection method).

Arguments:     None.

init:          Use to initialize the spectrometer.

Arguments:     None.

readmethod:    Use to load the data collection method specified

by the method_name property.

Arguments:     None.

ready:         Use to get the spectrometer setup.

Arguments:     None.

savemethod:    Use to save the current method.

Arguments:     None.

scan:          Use to collect a sample (using the current--most

recently loaded--data collection method).

Arguments:     None.

showobj:       Use to report all current settings for all bench

object properties.

Arguments:     None.

test:          Use to perform beamsplitter auto-alignment.

Arguments:     None.

###

___cmdargs_____________________________________________________________

Description: Cmdargs is an object that contains the read only

properties of command line arguments passed using the

run command.

Properties:  Properties for the cmdargs object are:

count:         The number of arguments passed to the current

macro.

Get syntax:    r = cmdargs:count

Returns:       (int).

type:          An array that contains the types of values

(integer, string, float, etc.) passed.

Arguments:     The passed argument's index in the array (int).

Get syntax:    r = cmdargs:type[index]

where "index" is a number between 1 and

cmdargs:count, inclusive.

Returns:       (string).  Valid return strings are: string, int,

float, byte, char, array and unknown.  (The return

value is "unknown" if an error occurred in passing

arguments.

args:          An array containing the actual values passed to

the macro.

Arguments:     The passed argument's index in the array (int).

Get syntax:    For all arguments except arrays:

r = cmdargs:args[index]

Returns:       The literal value that was passed. (The return

type is: cmdargs:type[index].)

For arrays:

userarray:data = cmdargs:args[index]

Result:        The userarray takes on all properties of the array

structure passed.

###

___com1___com2___com3___com4___________________________________________

Description: Four objects, com1, com2, com3 and com4, contain

properties and methods you can use to monitor and

control serial communications.  All four objects share

all of the same properties and methods, as described

below.  Error codes associated with these object are

also described below.

Properties:  NOTE: All properties, with the exception only of

verbose and errCount, should be set when the object

(the serial communication port) is closed.  If the port

is open, the return value is an error code

(-2, IE_OPEN) and the errCount property is not

incremented.

NOTE: Default property values are the settings put in

place at port initialization.

Properties for the com1, com2, com3 and com4 objects

are:

baud:          Use to set or get the baud rate for the object.

Arguments:     300, 600, 1200, 2400, 4800, 9600, 19200, 38400, or

57600.

Set Syntax:    r = com1:baud = 9600

Returns:       If successful r = 1, if not r = error code (int).

Get Syntax:    r = com1:baud

Returns:       The current baud rate for com1.

Default:       38400

dataBits:      Use to set or get the number of bits per

character.

Arguments:     A number from 4 to 8 (int).

Set Syntax:    r = com1:dataBits = 8

Returns:       If successful r = 1, if not r = error code (int).

Get Syntax:    r = com1:dataBits

Returns:       The current number of data bits for com1 (int).

Default:       8

stopBits:      Use to set or get the number of stop bits.

Arguments:     1 or 2 (int).

Set Syntax:    r = com1:stopBits = 1

Returns:       If successful r = 1, if not r = error code (int).

Get Syntax:    r = com1:stopBits

Returns:       The current number of stop bits for com1 (int).

Default:       1

parity:        Use to set or get the desired parity scheme.

Arguments:     A number to indicate parity (int), where 0 = none

1 = odd and 2 = even.

Set Syntax:    r = com1:parity = 0

Returns:       If successful r = 1, if not r = error code (int).

Get Syntax:    r = com1:parity

Returns:       The current parity scheme code for com1 (int).

Default:       0 (none)

flow:          Use to set or get the flow control scheme.

Arguments:     A number to indicate flow control (int), where

0 = none, 1 = CTS/RTS, 2 = DSR/DTR and

3 = XON/XOFF.

Set Syntax:    r = com1:flow = 1

Returns:       If successful r = 1, if not r = error code.

Get Syntax:    r = com1:flow

Returns:       The current flow control scheme for com1 (int).

Default:       1 (CTS/RTS)

rxBufferSize:  Use to set or get the receive buffer size.

Arguments:     Any value between 2048 and 32768, inclusive (int).

Set Syntax:    r = com1:rxBufferSize = 2048

Returns:       If successful r = 1, if not r = error code (int).

Get Syntax:    r = com1:rxBufferSize

Returns:       The current receive buffer size for com1 (int).

Default:       2048

txBufferSize:  Use to set or get the transmit buffer size.

Arguments:     Any value between 2048 and 32768, inclusive (int).

Set Syntax:    r = com1:txBufferSize = 2048

Returns:       If successful r = 1, if not r = error code (int).

Get Syntax:    r = com1:txBufferSize

Returns:       The current transmit buffer size for com1 (int).

Default:       2048

rxTimeOutVal:  Use to set or get the receive timeout value, in

seconds.

Arguments:     Any value between 1 and 60, inclusive (int).

Set Syntax:    r = com1:rxTimeOutVal = 1

Returns:       If successful r = 1, if not r = error code (int).

Get Syntax:    r = com1:rxTimeOutVal

Returns:       The current receive timeout value for com1 (int).

Default:       1

txTimeDelay:   Use to set or get the transmit delay value, in

seconds.

Arguments:     Any value between 0 and 60, inclusive (int).

Set Syntax:    r = com1:txTimeDelay = 0

Returns:       If successful r = 1, if not r = error code (int).

Get Syntax:    r = com1:txTimeDelay

Returns:       The current value for transmit delay between

characters for com1 (int).

Default:       0

errCount:      Use to retrieve the accumulated error count.

Arguments:     0 (resets errCount and lastError).

Set Syntax:    com1:errCount = 0

Returns:       Always sets r = 1 (int).

Get Syntax:    r = com1:errCount

Returns:       The number of errors on com1 since errCount was

reset (int).

lastError:     Use to get the last error encountered.

Arguments:     None.

Get Syntax:    r = com1:lastError

Returns:       The error code for last error encountered on com1

(int).  NOTE: The setting for the lastError

property is cleared when errCount is reset or when

the port is successfully opened.

verbose:       Use to enable or disable verbose operation.

Arguments:     1 to enable or 0 to disable (int).

Set Syntax:    com1:verbose = 1

Returns:       Always sets r = 1.

Get Syntax:    r = com1:verbose

Returns:       The current verbosity mode for com1 (int).

Default:       0 (disabled)

Methods:     Methods for the com1, com2, com3 and com4 objects are:

open:          Use to open the communications port.

Arguments:     1 (int).

Set Syntax:    r = com1:open = 1

Returns:       If successful r = 1, if not r = error code (int).

Get Syntax:    r = com1:open

Returns:       The current port open status of com1 (int).

close:         Use to close an open communications port.

Arguments:     1 (int).

Set Syntax:    r = com1:close = 1

Returns:       If successful r = 1, if not r = error code (int).

setRTS:        Sets the RTS (request-to-send) signal.

Arguments:     None.

Set Syntax:    r = com1:setRTS

Returns:       If successful r = 1, if not r = error code (int).

clearRTS:      Clears the RTS (request-to-send) signal.

Arguments:     None.

Set Syntax:    r = com1:clearRTS

Returns:       If successful r = 1, if not r = error code (int).

setDTR:        Sets the DTR (data-terminal-ready) signal.

Arguments:     None.

Set Syntax:    r = com1:setDTR

Returns:       If successful r = 1, if not r = error code (int).

clearDTR:      Clears the DTR (data-terminal-ready) signal.

Arguments:     None.

Set Syntax:    r = com1:clearDTR

Returns:       If successful r = 1, if not r = error code (int).

setXON:        Causes transmission to act as if XON character

has been sent.

Arguments:     None.

Set Syntax:    r = com1:setXON

Returns:       If successful r = 1, if not r = error code (int).

setXOFF:       Causes transmission to act as if XOFF character

has been sent.

Arguments:     None.

Set Syntax:    r = com1:setXOFF

Returns:       If successful r = 1, if not r = error code (int).

setBreak:      Causes transmission to suspend and port enters

break state.

Arguments:     None.

Set Syntax:    r = com1:setBreak

Returns:       If successful r = 1, if not r = error code (int).

clearBreak:    Restarts transmission and port exits break state.

Arguments:     None.

Set Syntax:    r = com1:clearBreak

Returns:       If successful r = 1, if not r = error code (int).

getRXcount:    Returns the number of characters in the receive

queue.

Arguments:     None.

Get Syntax:    r = com1:getRXcount

Returns:       If successful r = number of characters in the

queue, if not r = error code (int).

getTXcount:    Returns the number of characters in the transmit

queue.

Arguments:     None.

Get Syntax:    r = com1:getTXcount

Returns:       If successful r = number of characters in the

queue, if not r = error code (int).

flushRX:       Flushes any characters in the receive queue.

Arguments:     None.

Get Syntax:    r = com1:flushRX

Returns:       If successful r = 1, if not r = error code (int).

flushTX:       Flushes any characters in the transmit queue.

Arguments:     None.

Get Syntax:    r = com1:flushTX

Returns:       If successful r = 1, if not r = error code (int).

writeByte:     Use to transmit data out the selected port.

Arguments:     1 (int).

Set Syntax:    r = com1:writeByte = char

Returns:       If successful r = 1, if not r = error code (int).

writeString:   Use to transmit a null-terminated string out the

selected port.

Arguments:     The string to transmit (string).

Set Syntax:    r = com1:writeString = string

Returns:       If successful r = number of characters sent, if

not r = error code (int).

readByte:      Use to receive data from the selected port.

Arguments:     None.

Get Syntax:    r = com1:readByte

Returns:       If successful r = char, if not r = error code.

readBuffer:    Use to receive data into a buffer from the

selected port.

Arguments:     Number of characters to read (int).

Get Syntax:    buffer = com1:readBuffer = count

Returns:       If successful buffer is valid, if not buffer

= NULL.

msg:           Use to display brief port status in the Genesis

message area.

Arguments:     None.

Syntax:        com1:msg

Returns:       1 (int).

rpt:           Use to display complete port status in the Genesis

report area.

Arguments:     None.

Syntax:        com1:rpt

Returns:       1 (int).

errString:     Look up an error code and display its explanation

in the report area.  NOTE: The object used for

this method is irrelevant.

Arguments:     Error code (int).

Syntax:        string = com1:errString = -2

Returns:       The error string associated with the specified

error code.

helpProps:     Use to display brief description of object

properties in the report area.  NOTE: The object

used for this method is irrelevant.

Arguments:     None.

Syntax:        com1:helpProps

Returns:       1 (int).

helpCmds:      Use to display a brief description of object

methods in the report area.  NOTE: The object

used for this method is irrelevant.

Arguments:     None.

Syntax:        com1:helpCmds

Returns:       1 (int).

helpErrs:      Use to display brief a description of object

errors in the report area.  NOTE: The object used

for this method is irrelevant.

Arguments:     None.

Syntax:        com1:helpErrs

Returns:       1 (int).

Error Codes: NOTE: You can get an error code explanation using the

errString method, as described above.

Error codes for the com1, com2, com3 and com4 objects

are:

Code:  String:       Explanation:

-2     IE_OPEN       The requested port is already open.

-3     IE_NOPEN      The requested port is NOT open.

-4     IE_MEMORY     Memory allocation failure while creating

queues.

-5     IE_DEFAULT    Invalid port property was requested.

-10    IE_HARDWARE   A nonexistent port was requested.

-11    IE_BYTESIZE   An invalid number of data bits requested.

-12    IE_BAUDRATE   An invalid baud rate was requested.

-13    IE_CMDFAILED  The requested method (command) failed.

-14    IE_RXTIMEOUT  The port has timeout while receiving data.

-20    CE_RXOVER     A receive buffer overflow was detected.

-21    CE_OVERRUN    A receive character overrun was detected.

-22    CE_RXPARITY   A hardware parity error was detected.

-23    CE_FRAME      A hardware framing error was detected.

-24    CE_BREAK      A hardware break condition was detected.

-25    CE_CTSTO      CTS (clear-to-send) timeout during

transmission.

-26    CE_DSRTO      DSR (data-set-ready) timeout during

transmission.

-27    CE_RLSDTO     RLSD (receive-line-signal-detect) timeout

during transmission.

-28    CE_TXFULL     The transmit buffer is full.

-33    CE_MODE       An unsupported mode was requested.

###

___dde_________________________________________________________________

Description: The dde object contains methods for Dynamic Data

Exchange.  These methods are:

initiate:  Use to initiate, or open an application for Dynamic

Data Exchange.

Arguments: Channel number, from 1 to 16 (int), application

(string), system topic (string).

Syntax:    dde:initiate [channel number] [application] [topic]

Returns:   1 if successful, 0 if not (int).

Example:   r = dde:initiate 1 "EXCEL" "SYSTEM"

execute:   Use to execute a command within the specified channel.

Arguments: Channel number (int), command (string).

Syntax:    dde:execute [channel number] [command]

Returns:   1 if successful, 0 if not (int).

Example:   r = dde:execute 1 "[FILE.CLOSE(FALSE)]"

poke:      Use to send data to the specified channel.

Arguments: Channel number, from 1 to 16 (int), item (string),

data (any value).

Syntax:    dde:poke [channel number] [item] [value]

Returns:   1 if successful, 0 if not (int).

Example:   r = dde:poke 2 "R3C1" 25.5

request:   Use to retrieve data from an application.

Arguments: Channel number, from 1 to 16 (int), item to retrieve

(string).

Syntax:    dde:request [channel number] [item]

Returns:   Value requested if successful, 0 if not (int).

Example:   r = dde:request 2 "R3C1"

terminate: Terminate an open channel.

Arguments: Channel number, from 1 to 16 (int).

Syntax:    dde:terminate [channel number]

Returns:   1 if successful, 0 if not (int).

Example:   dde:terminate 2

###

___paper_______________________________________________________________

Description: Paper is an object that contains properties for

plotting.

Properties:  You can get the current setting for any property

specified by paper.  For example:

r = paper:template_name  /* Store current setting */

/* for the template_name */

/* property in register r. */

You can also set a new setting for any property

specified by paper.  For example:

paper:template_name = tmp  /* Set the setting for */

/* the template_name */

/* property to tmp. */

For the first example, the return value matches the

value of the property setting itself (integer, string,

etc.).  For the second example, the value you specify

must also match the value of the property setting.

Each property value is included with the property

description below.

Properties for the paper object are:

template_name: The name of the plot method (template) file.

Arguments:     The filename for the method (string).

Set syntax:    paper:template_name = filename

printer_index: The printer index.

Arguments:     The index value for the printer you want to use

(int).

Set syntax:    paper:printer_index = index

printer_port:  The port to which the printer is connected.

Arguments:     The index that represents the port (int), where

0 = COM1, 1 = COM2, 2 = LPT1, 3 = LPT2, 4 = file,

and 5 = bench (spectrometer).

Set syntax:    paper:printerport = index

orientation:   The orientation of the paper for plotting.

Arguments:     The index that represents the orientation (int),

where 1 = Portrait and 2 = Landscape.

Set syntax:    paper:orientation = index

paper_size:    The size of the paper used to plot.

Arguments:     The index that represents the paper size (int),

where 1 = letter, 3 = tabloid, 5 = Legal, 9 = A4,

12 = B5, and 13 = B4.

Set syntax:    paper:paper_size = index

units:         The units in which plot size is stated.

Arguments:     The index that represents the plotting units

(int), where 1 = inches and 2 = centimeters.

Set syntax:    paper:units = index

data_line_width: Width of the plotted data traces.

Arguments:     A number from 1 to 8 to represent relative line

width (int).

Set syntax:    paper:data_line_width = number

axes_line_width: Width of the plotted axes.

Arguments:     A number from 1 to 8 to represent the relative

width of axes lines (int).

Set syntax:    paper:axes_line_width = number

plot_elements: Specify whether to plot text, data, or both.

Arguments:     The index that represents the elements to plot

(int), where 1 = data, 2 = text and 3 = both.

Set syntax:    paper:plot_elements = index

data_area_offset_x: The amount to offset the left edge of the

plot from the left edge of the paper (given in

the units set with the units property).

Arguments:     A number to indicate the offset amount (float).

Set syntax:    paper:data_area_offset_x = number

data_area_offset_y: The amount to offset the top edge of the plot

from the top edge of the paper (given in the units

set with the units property).

Arguments:     A number to indicate the offset amount (float).

Set syntax:    paper:data_area_offset_y = number

data_area_size_x: The total width of the plotted data area

(given in the units set with the units property).

Arguments:     A number to indicate the plotted data area width

(float).

Set syntax:    paper:data_area_size_x = number

data_area_size_y: The total height of the plotted data area

(given in the units set with the units property).

Arguments:     A number to indicate the plotted data area height

(float).

Set syntax:    paper:data_area_size_y = number

report_area_offset_x: The amount to offset from the left edge of

the plotted report from the left side of the

paper (given in the units set with the units

property).

Arguments:     A number to indicate the offset amount (float).

Set syntax:    paper:report_area_offset_x = number

report_area_offset_y: The amount to offset the top of the plotted

report from the top edge of the paper (given in

the units set with the units property).

Arguments:     A number to indicate the offset amount (float).

Set syntax:    paper:report_area_offset_y = number

report_area_size_x: The total width of the plotted report area.

Arguments:     A number to indicate the plotted report width

(float).

Set syntax:    paper:report_area_size_x = number

report_area_size_y: The total height of the plotted report area

(given in the units set with the units property).

Arguments:     A number to indicate the plotted report height

(float).

Set syntax:    paper:report_area_size_y = number

printer_name:  The name of the printer to use.

Arguments:     The printer name (string).

Set syntax:    paper:printername = name

###

___parms_______________________________________________________________

Description: Parms is an object that contains properties and

methods you set and use to control several aspects of

the Genesis software environment.  All properties and

all methods for the parms object are described below.

Properties:  You can get the current setting for any property

specified by parms.  For example:

r = parms:bkgcolor  /* Store the current setting for */

/* the bkgcolor property in the */

/* r register. */

You can also set a new setting for any property

specified by parms.  For example:

parms:labelshadowon = 1  /* Set the setting for the */

/* labelshadowon property */

/* to 1. */

For the first example, the return value matches the

value of the property setting itself (integer, string,

etc.).  For the second example, the value you specify

must also match the value of the property setting.

Each property value is included with the property

description below.

NOTE: For properties with a Color argument, the color

is represented by a 24-bit integer (which is easy to

represent with hexadecimal numbers).  The color is

determined by setting percentages of blue, green, and

red.  For instance, 0xFF00FF means 100% blue, 0% green,

and 100% red.  The leftmost FF represents percentage of

blue, the center values (00) represent percentage of

green, and the rightmost FF represents percentage

of red.  So, 0xFF0000 = 100% blue only, 0x00FF00 = 100%

green only, and 0x0000FF = 100% red only.

Properties for the parms object are:

allowdrtswitch: Use to specify if data type conversion from the

diffuse reflectance data type is allowed.

Arguments:     1 = Yes, 0 = No (int).

auditlevel:    Use to turn audit trail use on and off.

Arguments:     0 = Off, 1 = On (int).

axescolor:     Use to set the axes color.

Arguments:     Color (int).

bkgcolor:      Use to set the background color for buttons

and etc. in the software.

Arguments:     Color (int).

bottom:        The minimum y-axis value to display (when set

scaling limits are in use.)

Arguments:     The y-axis value (float).

buttoneffect:  Use to show buttons as 2D or as 3D.

Arguments:     1 for 2D, 2 for 3D (int).

chart:         Use to set the name of the current functional

group correlation chart.

Arguments:     Name of chart (string).

cwd:           Use to set the current working directory.

Arguments:     The full directory path (string).

displaymode:   Use to set the data display mode.

Arguments:     The number that represents the display mode (int),

where 1 = Linear, and 2 = Expanded.

filetype:      The default data type to load.

Arguments:     The number that represents the data type (int),

where 0 = abs, 1 = ras, 2 = drt,

3 = sbm, 4 = bkg, 5 = igm and 6 = big.

fkeycolor:     Use to set the color for Fkey and tab

function key identifiers (F1, F2, etc. and Alt +

F1, Alt + F2, etc.).

Arguments:     Color (int).

grid:          Use to set the grid style.

Arguments:     The number that represents the grid style (int),

where 0 = no grid, 1 = line grid, 2 = cross grid,

and 3 = dot grid.

gridcolor:     Use to set the grid color.

Arguments:     Color (int).

hilitecolor:   Use to specify the highlight color (for a listed

method, etc.).

Arguments:     Color (int).

holecolor:     Use to set the color for the data display area

background.

Arguments:     Color (int).

labelcolor:    Genesis: Use to set the name color for Fkeys and

the active tab.  WinFIRST: Use to set the axes

and axes label color.

Arguments:     Color (int).

labelshadowon: Use to turn label shadows on and off.

Arguments:     0 = Off, 1 = On (int).

language:      Use to specify the current language file to use.

Arguments:     Language file name (string).

left:          Use to set the leftmost wavenumber displayed (when

set scaling limits are in use).

Arguments:     The wavenumber (float).

library:       Use to specify the default spectral library to use

for spectral library searches.

Arguments:     Library name (string).

lockouts:      Use to specify whether you want to lock out all

WinFIRST procedures that change program settings.

Arguments:     0 = Off, 1 = On (int).

logon:         Use to specify whether Genesis displays a log on

prompt as it starts.

Arguments:     1 = Yes, 0 = No (int).

macro:         Use to specify the name of the current macro.

(The current macro is, otherwise, the last macro

loaded using the Load Fkey within scan

administration macro operations.  Also, it is the

current macro that is run when the Run Macro

Fkey is selected from within scan administration

macro operations.)

Arguments:     The macro filename (string).  (The macro file

extension, .mac, is assumed and need not be

specified.)

mathMethod:    Use specify the current math method

(configuration).

Arguments:     Any valid math method file name (string).

Syntax:        parms:mathmethod = "special"

month:         Use to set the current month.

Arguments:     A number from 1 to 12 to correspond with one of

the 12 months (int).

monochrome:    Use to specify monochrome or color display.

Arguments:     1 for monochrome, 0 for color (int).

name:          Use to specify the name of this parms (parameter)

file.

Arguments:     The name of the file (string).

operator:      Use to specify the name of the current operator.

(This name is otherwise supplied as the operator

logs on as the Genesis software starts (when

operator log on is in use).  Also, it is the

current operator name that appears in the sample

data tagger (when taggers are in use and the

tagger is defined to include the operator name).

Arguments:     Operator name (string).

overlaystyle:  Use to set the data display style.

Arguments:     1, 2, 4, or 8 (int), where 1 = Stack, 2 = Overlay,

4 = Superimpose, and 8 = Sample Only.

overwriteprompt: Use to specify whether Genesis displays a prompt

before overwriting an existing file.

Arguments:     1 = Yes, 0 = No (int).

peakthreshold: Use to specify peak threshold, by specifying a

percentage of the data's maximum y value.  Only

those peaks above the defined peak threshold are

reported.

Arguments:     The percentage value (float).  For example 0.25

specifies a threshold value of 25 percent.

plotformat:    Use to specify the name of the plot method to use

as the current method.

Arguments:     Plot method file name (string).

plotMethod:    Use specify the current plot method

(configuration).

Arguments:     Any valid plot method file name (string).

Syntax:        parms:plotmethod = "special"

ref1color:     Use to set the R1 (first) reference color.

Arguments:     Color (int).

ref2color:     Use to set the R2 (second) reference color.

Arguments:     Color (int).

ref3color:     Use to set the R3 (third) reference color.

Arguments:     Color (int).

ref4color:     Use to set the R4 (fourth) reference color.

Arguments:     Color (int).

reportcolor:   Use to set the color for report text.

Arguments:     Color (int).

right:         Use to set the rightmost wavenumber displayed

(when set scaling limits are in use).

Arguments:     The wavenumber (float).

rptbkgcolor:   Use to set the color for the report area

background.

Arguments:     Color (int).

samplecolor:   Use to change the sample data color.

Arguments:     Color (int).

saveprompt:    Use to specify whether Genesis displays a prompt

before discarding changed data.

Arguments:     1 = Yes, 0 = No (int).

scaling:       Use to set the data scaling method.

Arguments:     The number that represents the scaling method

(int), where 1 = autoscale, and 2 = use set

scaling limits.  (These limits are set using the

left, right, top and bottom parameters.)

scanMethod:    Use to specify the name of the data collection

method to use as the current method.

Arguments:     Method name (string).

Syntax:        parms:scanmethod = "special"

shadowcolor:   Use to set shadow color (inactive tab backgrounds,

Fkey shadows, etc.).

Arguments:     Color (int).

signon:        Use to specify whether you want to display the

WinFIRST Log Book dialog each time WinFIRST is

started and require entry of log book record.  If

the operator chooses Cancel from the Log Book

dialog to avoid entering a log book record,

WinFIRST closes.

Arguments:     0 = Off, 1 = On (int).

simplemenus:   Use to specify whether you want WinFIRST to start

in simple menus mode.

Arguments:     0 = No, 1 = Yes (int).

smooth_method: Use to set the smooth method to use for spectral

smoothing.

Arguments:     The number that represents the smooth methods

(int), where 0 = Boxcar, 1 = Cubic, 2 = Quartic,

3 = Quadratic, and 4 = Quintic.

smooth_points: Use to specify the number of smoothing points to

use for spectral smoothing.

Arguments:     Any odd number from 3 to 25 (int).

stacksize:     Use to set the number of displayed data files (one

sample file and up to four reference files).

Arguments:     A number from 1 to 5 to specify the total number

of data files to display.

startinadminmode: Use to specify whether Genesis starts in

administrator mode.

Arguments:     1 = Yes, 0 = No (int).

tabcolor:      Use to set the name color for the inactive tab

keys.

Arguments:     Color (int).

tagger:        Use to specify whether a sample data tagger is

used.

Arguments:     1 = Yes, 0 = No (int).

tagstring:     Use to define the current tagger string.

Arguments:     Tagger string (string).  NOTE: Refer to Chapter 8

of the Genesis Software Administrator's Guide for

a listing of the codes used in a tagger string.

testlevel:     Use to set the scan test level.

Arguments:     The number that represents the test level (int),

where 0 = standard test, 1 = interferogram test,

2 = linearity and water test, and 3 = 100% line

noise test.

top:           Use to set the maximum y-axis value to display

(when set scaling limits are in use).

Arguments:     The y-axis value (float).

xdigits:       Use to set the number of decimal places to include

in reported x-axis values.

Arguments:     0 to 9 (int), where 0 specifies no decimal

places and 9 specifies 9 decimal places.

ydigits:       Use to set the number of decimal places to include

in reported y-axis values.

Arguments:     0 to 9 (int), where 0 specifies no decimal

places and 9 specifies 9 decimal places.

Methods:     Methods for the parms object are:

read:          Use to load a Genesis parms file.  (Set the

parms file name with "parms:name")

Arguments:     None.

Example:       parms:read  /* Read in the parms file. */

write:         Use to save a Genesis parms file.  (Set the

parms file name with "parms:name".)

Arguments:     None.

Example:       parms:write  /* Save in the parms file. */

###

___quant_______________________________________________________________

Description: Quant is an object that contains properties and

methods you can set and execute to perform

quantitative analysis.

Properties:  Properties for the quant object are:

method:        Used to set the name of the current quant method

file that contains xy pairs (string).  NOTE: This

file must be in ASCII format with one xy pair per

line.  NOTE: The method property for the quant

object is the same as the calibrate command.

Arguments:     The name of the file that contains the method

information (string).

Syntax:        quant:method = "filename"

Methods:     Methods for the quant object are:

predict:       Used to perform the quantitative analysis based

on the calibrated method.  NOTE: The predict

method for the quant object is the same as

the predictY command.  The predict method returns

the floating point value predicted using the y =

mx + b linear relationship established by the

calibration.

Arguments:     None.

Syntax:        quant:predict

Example:       /* Load a method and do a calibration: */

quant:method = "c:\first\data\quant\xy.dat"

/* Find an absorbance value of interest: */

p = peakAt 3100

/* Predict a concentration from the absorbance: */

c = quant:predict p

print "Concentration = " c

slope          Returns the (floating point) slope from the

calibration.

Arguments:     None.

Syntax:        quant:slope

intercept:     Returns the (floating point) intercept from the

calibration.

Arguments:     None.

Syntax:        quant:intercept

correlation    Returns the (floating point) correlation from

the calibration.

Arguments:     None.

Syntax:        quant:correlation

stddev         Returns the (floating point) standard deviation

from the calibration.

Arguments:     None.

Syntax:        quant:stddev

###

___reference___________________________________________________________

Description: Reference is an object that contains properties and

methods you can set and execute to perform various

operations with the R1 (first) reference.

Properties:  Properties for the reference object are:

name:          The reference file name.

Arguments:     If setting the name, specify the name (string).

Set syntax:    reference:name = "name"

Get syntax:    r = reference:name

Get returns:   (string).

title:         The title for the current reference.

Arguments:     If setting the title, specify the title (string).

Set syntax:    reference:title = "title"

Get syntax:    r = reference:title

Get returns:   (string).

points/rows:   The number of data points.  (This is the same as

the number of rows in data array).

Arguments:     None.  (retrievable only)

Get syntax:    r = reference:points

Get returns:   (int).

firstx:        The starting wavenumber of the spectrum.

Arguments:     None.  (retrievable only)

Get syntax:    r = reference:lastx

Get returns:   (float).

lastx:         The ending wavenumber of the spectrum.

Arguments:     None.  (retrievable only)

Get syntax:    r = reference:lastx

Get returns:   (float).

start:         The starting index of the data array.

Arguments:     None.  (retrievable only)

Get syntax:    r = reference:start

Get returns:   (int).

data:          Use for direct access to the reference data.

Set syntax:    To set an individual data point:

reference:data[point] = value

where, "point" is an index between 0 and

(reference:points - 1), inclusive

and "value" is the y value at the specified index

(float).

To set entire data array:

reference:data = userarray:data

where, "userarray" is a user-defined array.

Get syntax:    To retrieve an individual data point:

r = reference:data[point]

where, "point" is an index between 0 and

(reference:points - 1), inclusive.

To retrieve entire data array:

userarray:data = reference:data

where, "userarray" is a user-defined array.

NOTE: userarray acquires every property of the

reference array.

Methods:     Methods for the reference object are:

clear:         Clear current reference.

Arguments:     None.

Syntax:        reference:clear

clearall:      Clears all reference spectra from data window.

Arguments:     None.

Syntax:        reference:clearall

load:          Load current file (name set by reference:name).

Arguments:     None.

Syntax:        reference:load

numrefs:       Use to return the number of reference spectra in

the data window.

Arguments:     None.

Syntax:        reference:numrefs

overlay:       Use to overlay the references and the sample.

Arguments:     None.

Syntax:        reference:overlay

rolldown:      Roll the reference stack down one position.

(Works the same as the related toolbar icon.)

Arguments:     None.

Syntax:        reference:rolldown

rollup:        Roll the reference stack up one position.

(Works the same as the related toolbar icon.)

Arguments:     None.

Syntax:        reference:rollup

stack:         Use to stack the references and the sample.

Arguments:     None.

Syntax:        reference:stack

superimpose:   Use to superimpose the references and the sample.

Arguments:     None.

Syntax:        reference:superimpose

###

___report______________________________________________________________

Description: Report is an object that contains properties and

methods you can set and execute to perform various

operations with the Genesis report area.

Properties:  Properties for the report object are:

name:          Use to set the name (filename) for the report.

Arguments:     The name of the file (string).

Syntax:        report:name = "filename"

top:           Use to set and retrieve the line number of the

top of the reports area.

Arguments:     (optional) Line number to set to top of report.

Set syntax:    report:top 12  /* Set line 12 to top of report. */

Get syntax:    r = report:top  /* Put top line number in r. */

Get returns:   (int).

Methods:     Methods for the report object are:

clear:         Use to clear the Genesis report area.

Arguments:     None.

Syntax:        report:clear

iconize:        (WinFIRST only.)  Minimizes the active report

window to an icon.  (Same as the minimize method

for the report object.

Arguments:      None.

Syntax:         report:minimize

maximize:       (WinFIRST only.)  Maximizes the active report

window to full size.

Arguments:      None.

Syntax:         report:maximize

minimize:       (WinFIRST only.)  Minimizes the active report

window to an icon.  (Same as the iconize method

for the report object.)

Arguments:      None.

Syntax:         report:minimize

nextWindow:    (WinFIRST only.)  Use to make the next report

window in the window list the active window.

Arguments:     None.

Syntax:        report:nextwindow

new:           (WinFIRST only.)  Use to create a new report

window.

Arguments:     None.

Syntax:        report:new

numWindows:    (WinFIRST only.)  Use to return the number of

open report windows.

Arguments:     None.

Syntax:        report:numwindows

read:          Use to read the report identified by the name

property (and stored on disk) into the Genesis

report area.

Arguments:     None.

Syntax:        report:read

show:          Redraw the active report window.  If the window is

iconized, it is returned to it previous size.  If

the window is behind other windows, it is

brought to the top.

Arguments:     None.

Syntax:        report:show

write:         Uses to write (save) the current report text in

the Genesis report area as a file.  The name set

by the name property is automatically assigned to

the report file that is created.

Arguments:     None.

Syntax:        report:write

###

___sadtler2matt________________________________________________________

Description: Use to a translate a copy of a file in the Sadtler

format (.isp or .irf) to a file in the ATI Mattson

FIRST data file format.  The translated file becomes

the current sample.  The original Sadtler file that

is translated is not removed or otherwise altered by

the sadtler2matt command.  You can use the translated

data file in the same way you would any data file

created by WinFIRST.

Arguments:   The directory path and name of Sadtler file (string).

Example:     sadtler2matt "\sadtler\data\pstyrene"

/* Translate file and make it the */

/* sample data file. */

###

___sample______________________________________________________________

Description: Sample is an object that contains properties and

methods you can set and execute to perform various

operations on the current sample data.

Properties:  Properties for the sample object are:

audit:          Use to add text you specify to the sample data

file audit trail.  If no audit trails exists, one

is created.

Arguments:      The text string to add, placed within double

quote marks (string).

data:           Use for direct access to the sample data.

Set syntax:     To set an individual data point:

sample:data[point] = value

where, "point" is an index between 0 and

(sample:points - 1), inclusive

and "value" is the y value at the specified index

(float).

To set an entire data array:

sample:data = userarray:data

where, "userarray" is a user-defined array.

Get syntax:     To retrieve an individual data point:

r = sample:data[point]

where, "point" is an index between 0 and

(sample:points - 1), inclusive

To retrieve entire data array:

userarray:data = sample:data

where, "userarray" is a user-defined array.

NOTE: userarray acquires every property of the

sample array.

firstx:         The starting wavenumber of the sample spectrum.

Arguments:      None.  (retrievable only)

Get syntax:     r = sample:lastx

Get returns:    (float).

lastx:          The ending wavenumber of the sample spectrum.

Arguments:      None.  (retrievable only)

Get syntax:     r = sample:lastx

Get returns:    (float).

name:          The file name of the current sample.

Arguments:     If setting the name, specify the name (string).

Set syntax:    sample:name = "name"

Get syntax:    r = sample:name

Get returns:   (string).

points/rows:   The number of data points.  (This is the same as

the number of rows in the data array.)

Arguments:     None. (retrievable only)

Get syntax:    r = sample:points

Get returns:   (int).

start:          The starting index of the data array.

Arguments:      None.  (retrievable only)

Get syntax:     r = sample:start

Get returns:    (int).

title:         The title of the current sample.

Arguments:     If setting the title, specify the title (string).

Set syntax:    sample:title = "title"

Get syntax:    r = sample:title

Get returns:   (string).

xAxisLabel:    Use to set the x axis label for the current data

window.

NOTE: The display is not changed until you use the

show method for the sample object, as described

below.

Arguments:     The label (string) to use.

Set syntax:    sample:xaxislabel = "Concentration"

yAxisLabel:    Use to set the y axis label for the current data

window.

NOTE: The display is not changed until you use the

show method for the sample object, as described

below.

Arguments:     The label (string) to use.

Set syntax:    sample:yaxislabel = "Arbitrary Units"

Methods:     Methods for the sample object are:

clear:          Clear (unload) current sample.

Arguments:      None.

Syntax:         sample:clear

copy:           (WinFIRST only.)  Copy the current sample data

to the WinFIRST clipboard.

Arguments:      None.

Syntax:         sample:clear

iconize:        (WinFIRST only.)  Minimizes the active data

window to an icon.  (Same as the minimize method

for the sample object.

Arguments:      None.

Syntax:         sample:iconize

Example:        if (sample:points)

{

a = sample:numwindows

for i = 1 to a step 1

{

sample:iconize

}

}

load:           Load specified file (name set by sample:name).

Arguments:      None

Syntax:         sample:load

maximize:       (WinFIRST only.)  Maximizes the active data

window to full size.

Arguments:      None.

Syntax:         sample:maximize

minimize:       (WinFIRST only.)  Minimizes the active data

window to an icon.  (Same as the iconize method

for the sample object.  See the iconize method

description for an example.)

Arguments:      None.

Syntax:         sample:minimize

nextWindow:     (WinFIRST only.)  Makes the next data window in

the window list the active data window.

Arguments:      None.

Syntax:         sample:nextWindow

numWindows:     (WinFIRST only.)  Returns the number of data

windows currently open.  This method is useful

for looping through all current samples.

Arguments:      None.

Syntax:         sample:numWindows

Example:        if (sample:points)

{

a = sample:numwindows

sample:maximize

for i = 1 to a step 1

{

pause "Press Enter to view next sample"

sample:nextWindow

}

}

pasteAsReference:  (WinFIRST only.)  Paste the data currently on

the WinFIRST clipboard to the data window as a

reference data file.  If no data window is open,

a new window is opened and the clipboard data is

pasted to that window as sample data.

Arguments:      None.

Syntax:         sample:pasteAsReference

pasteAsSample:  (WinFIRST only.)  Paste the data currently on

the WinFIRST clipboard to the data window as the

sample data file.  If no data window is open,

a new window is opened and the clipboard data is

pasted to that window as sample data.

Arguments:      None.

Syntax:         sample:pasteAsSample

reload:         Reload current sample.

Arguments:      None.

Syntax:         sample:reload

reportAudit:    Places the audit trail report for the current

sample data in a new report window.

Arguments:      None.

Syntax:         sample:reportAudit

show:           Redraw the active data window.  If the window is

iconized, it is returned to it previous size.  If

the data window is behind other windows, it is

brought to the top.

Arguments:      None.

Syntax:         sample:show

toBackground:   Make the current sample the current background

data.

Arguments:      None.

Syntax:         sample:toBackground

toclipboard:    Place current sample on Windows clipboard.

Arguments:      None.

Syntax:         sample:toclipboard

###

___any expression_____________________________________________________

Description: Use to enter any ApPro built-in macro command.  You

should have a good working knowledge of ApPro

commands before using this advanced feature.

###

___fft________________________________________________________________

Description: FFT:process is used to perform a simple Fast

Fourier Transform on the current interferogram

(.igm) sample.

###

___md_________________________________________________________________

Description: Use to create a new directory.  This command

works the same as the DOS MD command.  NOTE: you can

create only one directory at a time.

Arguments:   Drive and directory path.

Example:     md c:\genesis\data\newdata  /* Make a directory named */

/* 'newdata' in the */

/* '\genesis\data' */

/* directory. */

###

___if________________________________________________________________

Description: Use to test the truth of an expression, and

act according to the results.  If the expression

evaluates to true, the body of code contained

in the block following the 'if' statement is

executed, otherwise it is not.  An 'else'

statement can follow immediately after the 'if'

code block.  In this case, when the 'if' code

block is not executed, the 'else' code block is

executed.

Example 1:   if (A > 3)

begin              /* Start of 'if' code block. */

load "tmp.abs"   /* If A > 3, load tmp.abs file. */

end                /* End of 'if' code block. */

/* Otherwise, do nothing. */

Example 2:   if (A > 3)

begin              /* Start of 'if' code block. */

load "tmp.abs"  /* If A > 3 load tmp.abs file.*/

end                /* End of 'if' code block. */

else

begin              /* Start of 'else' code block. */

load "pure.abs"  /* If A <= 3, load file. */

end                /* End of 'else' code block. */

###

___while_____________________________________________________________

Description: Use to test the truth of an expression, and

act according to the results.  As long as the

expression is evaluated to true, the code will

be executed.  When the code in the 'while' block

has been executed, control is returned to the

'while' statement.  If the expression is

evaluated to true, the code block is executed

again.  This continues until the expression

evaluates to false, or a loop break is used.

Example:     A = 0

while (A < 2)     /* If A < 2, load a reference. */

begin             /* Start of 'while' block. */

loadref         /* Load a reference. */

A = A + 1       /* Increment the value of A. */

end               /* End of 'while block. */

###

___choice___________________________________________________________

Description: Use to ask the user a yes or no question and

act according to the results. If the user

answers yes, the body of code contained

in the block following the 'if' statement is

executed, otherwise it is not.

Example:     if(ask "would you lik to load a sample")

begin               /* Start of 'if' code block. */

load              /* If yes, load a sample. */

end                 /* End of 'if' code block. */

/* Otherwise, do nothing. */

###

___choice and branch_________________________________________________

Description: Use to ask the user a yes or no question and

act according to the results.  If the user

answers yes, the body of code contained

in the block following the 'if' statement is

executed, otherwise the body of code contained

in the block following the 'else' statement is

executed.

Example:     if(ask "would you lik to load a sample")

begin          /* Start of 'if' code block. */

load         /* If yes, load a sample. */

end            /* End of 'if' code block. */

else

begin          /* Start of 'else' code block. */

print "nothing loaded"  /* If no, print message. */

end            /* End of 'else' code block. */

###

___decision loop______________________________________________________

Description: Use to ask the user a yes or no question and

act according to the results.  As long as the

user answers yes to the question, the code will

be executed.  When the code in the 'while' block

has been executed, control is returned to the

'while' statement.  If the user answers yes to the

question again, the code block is executed

again.  This continues until the user answers

no to the question.

Example:     A = 0

while (ask "do you want to load a reference")

begin               /* Start of 'while' block. */

loadref           /* Load a reference. */

end                 /* End of 'while' block. */

###

___math expression____________________________________________________

Description: Use this to enter an ApPro math expression. You should

have a good working knowledge of ApPro math commands

before using this advanced feature.

###

___break_____________________________________________________________

Description:  Use to immediately halt execution of a loop.

Example:   A = 0

while(A < 10)   /* Allow up to 10 executions. */

begin

if (ask "load another reference?")

begin

loadref      /* Load a reference. */

A = A + 1    /* Increment the value of A. */

end

else           /* If answer was 'no' then */

break;       /* stop execution of the loop. */

end

###

___loadref_____________________________________________________________

Description: Use to open (or load) a data file into memory

and display the data as reference data.

Arguments:   The name of the file you want to load.  If the file

is not in the current directory, you must specify

the complete file path (including drive designation).

Example:     loadref "c:\genesis\data\standard\pure1.abs"

/* Load the file into memory */

/* and display it in the data */

/* display area as a reference. */

###

___scaleToPeak________________________________________________________

Description: Use to scale the reference data to the y value of

the sample peak which is closest to the supplied

wavenumber.

Arguments:   The wavenumber at which to find the peak.

Example:     load "pstyrene.abs"  /* Load file into memory. */

loadref "pure1.abs"  /* Load reference. */

scaleToPeak 2250     /* Scale to peak at 2250. */

###

___fwhm_______________________________________________________________

Description: Use to retrieve the peak width at half the absorbance

at the sample peak closest to the supplied

wavenumber.

Arguments:   The wavenumber at which to find the peak.

Example:     load "pstyrene.abs"  /* Load file into memory. */

fwhm 2250            /* Scale to peak at 2250. */

###