- Transfer arm requirements ≥46" linear travel of arm.

- Overall length of outer housing ≤ 65"?

- length of arm extending from CF flange to center of Stub mounted on fork  7.25" (including fork)

- Fork length 2" from end of outer arm housing to center of Stub.

- Length of arm extending from CF flange to end of outer arm 5.25"

- camshaft on end of arm to operate Service Physics Fork with 1/2" cam (see Service Physics dimensional drawing of Fork below and website http://mmrc.caltech.edu/mmrc_html/XPS/XPS-Manipulator/XPS-Manipulator.html)

- Flat or adaptor to mount Service Physics Fork mount (see Service Physics dimensional drawing of Fork below also see website http://mmrc.caltech.edu/mmrc_html/XPS/XPS-Manipulator/XPS-Manipulator.html)  Fork currently approximately 11.5 mm below center line of 2 ¾" CF.

- 2 ¾" CF flange to attach to system.

- 5mm inner shaft continuous 360 rotation

- extended handle to allow operator to position himself/herself to observe end of arm.

- extension handle will not be able to rotate inner shaft

- Ability to easily rotate inner shaft when necessary with 1:1 rotation of control magnet and inner shaft

- bakeout to 250 C

- port aligner.

All dimensions approximate.  Will supply exact drawing for construction.

Top view of Fork.

Side view of Fork.